# VPNs for International Service Providers

PoC Template #2

| Last Updated: | Dec 25, 2014 |
| --- | --- |
| Doc. Version: | 1.0 |
| Doc. Owner | Sarit Carmeli |

### Contents

1.	Introduction	3

1.1.	Purpose	3

1.2.	Scope	3

1.3.	Layout Overview	3

1.3.1.	Application Diagram	3

1.3.2.	Devices under Test	3

1.3.3.	Devices Under Test	3

2.	Preliminary Configuration and Management	3

2.1.	Devices Preliminary Setting	3

2.1.1.	ETX-5 Preliminary Setting	3

2.1.2.	ETX-2 Preliminary Setting	3

2.2.	RADview Preliminary Setting	3

2.2.1.	Define Business Entities	3

2.2.2.	Define the Policies	3

3.	Building the RADview Map	3

3.1.	Define the catalog	3

3.1.1.	Define the CoS: Priority, p-bit, Queue	3

3.1.2.	Define the Ethernet bandwidth	3

3.1.3.	Define the service catalog	3

3.1.4.	Define the Test Catalog	3

3.2.	Add the entities (ETXs and Cloud)	3

3.3.	Create Links between the Devices	3

3.4.	Define the Tunnels	3

3.5.	Define the Trap Sync and Alarm Sync	3

3.6.	Define Statistics Collection Job	3

4.	Service Provisioning and Testing	3

4.1.	Backbone – E-Line over Tunnel Services between Edge Endpoints	3

4.1.1.	Test Case 1: Single CoS; New York to Tokyo; Add&Pop	3

4.1.2.	Test Case 2: Single CoS; London to Tokyo; Add&Pop	3

4.1.3.	Test Case 3: Multi CoS; New York to Tokyo; No Manipulation	3

4.1.4.	Test Case 4: Multi CoS; London to Tokyo; No Manipulation	3

4.1.5.	Test Case 5: Multi CoS; New York to Tokyo; Add&Pop	3

4.1.6.	Test Case 6: Multi CoS; New York to Tokyo; Add&Keep	3

4.1.7.	Test Case 7: Multi CoS; London to Tokyo; Add&Pop	3

4.2.	Backbone –E-Line over Tunnel Services between Aggregation Devices	3

4.2.1.	Test Case 8: Single CoS; ETX-5_C to ETX-5_A; Add&Pop	3

4.2.2.	Test Case 9: Single CoS; ETX-5_A to ETX-5_B; Add&Swap	3

4.2.3.	Test Case 10: Multi CoS; ETX-5_B to ETX-5_C; Add&Keep	3

4.3.	Backbone – E-LAN over Tunnel services	3

4.3.1.	Test Case 11: E-LAN; Single CoS; Tokyo to New York and to London; Swap C.VLAN	3

4.3.2.	Test Case 12: E-LAN; Single CoS; Tokyo to New York and to London; Add Service VLAN	3

4.4.	Local Carrier – E-Line services	3

4.4.1.	Test Case 13; Single CoS; Tokyo to ETX-5_A; Add&Pop	3

4.4.2.	Test Case 14; Single CoS; London to ETX-5_C; Add&Pop	3

4.4.3.	Test Case 15; Single CoS; New York to ETX-5_B; No Manipulation	3

4.4.4.	Test Case 16; Single CoS; Tokyo to ETX-5_A; Add&Swap	3

4.4.5.	Test Case 17; Single CoS; New York to ETX-5_B; Add&Swap	3

4.4.6.	Test Case 18; Multi CoS; Tokyo to ETX-5_A; No Manipulation	3

4.4.7.	Test Case 19; Multi CoS; London to ETX-5_C; No Manipulation	3

4.4.8.	Test Case 20; Multi CoS; Tokyo to ETX-5_A; Add&Pop	3

4.4.9.	Test Case 21; Multi CoS; ETX-5_B to New York; Add&Keep	3

4.4.10.	Test Case 22; Multi CoS; London to ETX-5_C; Add&Keep	3

# Introduction

## Purpose

This document demonstrates the proper functionality of a generic Carrier Class Ethernet application based on the RAD ETX product line.

RAD’s ETX-A line of Carrier Ethernet demarcation and aggregation devices, together with the RADview management system will enable carriers and service providers to provide managed Carrier Ethernet connectivity solutions to their business and mobile client bases.

We have detailed and grouped our solutions for delivering EAD, EAD with timing capabilities and 10GbE EAD services together with the management and performance monitoring capabilities that are pivotal to meeting exacting business customer service expectations.

For the EAD standard  of Ethernet Service and un-contended port based product, the ETX-203A (available in both fixed and modular versions) will deliver all the features with advanced OAM, while the ETX-220 and ETX-5300A support 10GbE and beyond with high levels of efficiency and aggregation. The feature-rich, intelligent demarcation devices incorporate RAD’s own Carrier Ethernet ASIC to deliver SLA-based connectivity at the customer premises at a competitive price point.

All devices are fully equipped with a wealth of tools for sophisticated service delivery, traffic management and performance measurement. These significantly reduce service provisioning time, ensure high availability and resilience, and provide end-to-end Quality of Service for SLA assurance.

The ETX product portfolio offers a comprehensive offering of products enabling a single vendor access network solution for all for all Ethernet carrier needs.

## Scope

## Layout Overview

### Application Diagram

### Devices under Test

| ETX-203AX A low cost, high performance NTE delivering SLA-based Ethernet business services 2 x 1GbE NNI and 4 x 1 GbE UNI |  |
| --- | --- |
| ETX-205A High performance NTE delivering SLA-based Ethernet business services including Synchronous Ethernet, IEEE1588-2008, 2 x 1GbE ports in NNI and 4 x 1GbE port in UNI |  |
| ETX-220A 10G NTE delivering SLA-based EVPL services to the customer premises over native Ethernet access |  |
| ETX-5300A Aggregates up 80 FE/GE links from remote NTEs with advanced traffic management; highly accurate, hardware-based OAM and performance monitoring; 4 x 10GbE interfaces in the main module; power dissipation up to 500W; form-factor, 3U modular system. |  |

### Devices Under Test

The following tables contain the elements used in this PoC testing. It provides the elements name as well as ordering option and SW version was running during testing.

##### RAD POC Proposal BOM

| Product | Ordering Option | Description | QTY | SW Version |
| --- | --- | --- | --- | --- |
| ETX-5300 | ETX-5300A/AC | 5700040000 | 3 | 2.2.0(0.21) |
|  | ETX-5300A-MC/4XFP/AT | 6170210000 | 3 |  |
|  | ETX-5300A-ETH/20XGE/SFP | 6170060000 | 3 |  |
| ETX-205A | ETX-205A/AC/19V/QC18X/250 | 5271240000 | 1 | 5.7.0(0.55) |
| ETX-203AX | ETX-203AX\2SFP\4UTP | 6660020000 | 1 | 5.7.0(0.55) |
| ETX-220A | ETX-220A/AC/2XFP/20S/SYE/BSK | 5600470000 |  | 5.7.0(0.55) |
| SFP | SFP-6 | 1278050000 | 12 |  |
|  | XFP-1D | 1279010000 | 4 |  |
| RADVIEW | RADVIEW-PC/PMSM/DEMO | 5760260000 | 1 | 4.0.0(0.802) + ems40-v3.9.5 Patch |
|  | RV-LIC/ENW |  |  |  |

##### Test Equipment

| Function | Unit | Notes |
| --- | --- | --- |
| ETH Generator | Spirent |  |
| Fiber Cable | SM Fiber for SFP-6 and XFP-1D |  |
| Terminal Configuration Cable | RAD CBL-DB9-RJ45 | Supplied |

# Preliminary Configuration and Management

## Devices Preliminary Setting

Creating the services between the different locations will be done via RADview. However there are several parameters which need to be configured via CLI before beginning to work with RADview.

Before start with configuring of the ETX-2 and ETX-5, each should be set to its default parameters.

### ETX-5 Preliminary Setting

Access each of the ETX-5 units and verify that the ETX-5300A-ETH/20XGE/SFP module appears as the actual installed module in slot 1 (Configuration> show config cards-summary)

Configure each ETX-5 according to the scripts in Appendix A:

Note: each of the scripts should be modified according to the IP address of the unit, RADview and Default Gateway.

ETX-5_A HW, Queues, Port and Management Configuration scripts – 
refer to  Appendix A.1

ETX-5_B HW, Queues, Port and Management Configuration scripts – 
refer to  Appendix A.2

ETX-5_C HW, Queues, LAG, Port and Management Configuration scripts – 
refers to  Appendix A.3.

After running the scripts, verify the following:

The provisioned I/O modules are updated properly (Configuration> show config cards-summary).

The status of the LAG is UP, one of the ports is Active and LACP is Sync (Configuration>port>lag 1>show status)

Ping replies from the Management station towards each of the ETX-5.

Save the configuration for each of the ETX-5 devices.

### ETX-2 Preliminary Setting

- Configure each ETX-2 according to the scripts in appendix B:

Note: Each of the scripts should be modified according to the IP address of the unit, RADview and Default Gateway.

ETX-205A (Tokyo) Queues and Management Configuration scripts –
refer to  Appendix B.1

ETX-203AX (New York) Queues and Management Configuration scripts – 
refer to  Appendix B.2

ETX-220A (London) Queues and Management Configuration scripts 
refer to  Appendix B.3.

After running the scripts, verify ping replies from the management station towards each of the ETX-2 units.

Save the configuration for each ETX-2.

## RADview Preliminary Setting

### Define Business Entities

Create two Business Entities: Service Provider for the Clouds and Tunnels and Customer for the Services.

### Define the Policies

Define a new policy to determine the SLA parameters that the service will have to meet:

Availability

Frame Loss Ratio (FLR)

Average Frame Delay

Average Frame Delay Variation

# Building the RADview Map

## Define the catalog

### Define the CoS: Priority, p-bit, Queue

Define the CoS Catalog for Platinum, Gold and Best Effort Services, according to the following parameters:

| CoS Catalog |  |  |  |
| --- | --- | --- | --- |
| CoS Name | p-bit | Queue | p-bit Marking |
| Platinum | 5 | 0.Strict | No Color Marking |
| Gold | 2 | 4.WFQ |  |
| Best Effort | 1 | 5.WFQ |  |
| NO_Data_1 | 7 | 7.WFQ |  |
| NO_Data_2 | 7 | 7.WFQ |  |
| NO_Data_3 | 7 | 7.WFQ |  |
| NO_Data_4 | 7 | 7.WFQ |  |
| NO_Data_5 | 7 | 7.WFQ |  |

### Define the Ethernet bandwidth

Define the ETH BW Catalog according to the following parameters:

| BW Catalog |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | CIR (Kbps) | EIR (Kbps) | CBS (bytes) | EBS (bytes) |
| CIR=10M_EIR=0M | 10000 | -- | 128000 | -- |
| CIR=20M_EIR=0M | 20000 | -- | 128000 | -- |
| CIR=1M_EIR=9M | 1000 | 9000 | 128000 | 128000 |
| CIR=9M_EIR=0M | 9000 | -- | 128000 | -- |
| EIR=10M | -- | 10000 | -- | 12800 |
| Shaper=12M_CIR | 12000 | -- | 128000 | -- |

### Define the service catalog

- Define the service catalog that will determine the VLAN manipulation and the OAM parameters for  the E-Line services between Edge End Points according to the following table:

| # | Service Catalog Name | Service Type |  | Classification |  | CoS | BW |  | OAM & PM | Service Vlan Manipulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Policer | Shaper |  |  |
| 1 | 1_Single_Vlan_100 |  | Single CoS |  | Vlan+p-bit (100,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Pop |
| 2 | 2_Single_Vlan_300 |  | Single CoS |  | Vlan+p-bit (300,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Pop |
| 3 | 3_Multi_Vlan_400 |  | Multi CoS |  | Vlan+p-bit (400,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | No Manipulation |
|  |  |  |  |  | Vlan+p-bit (400,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  |  | Vlan+p-bit (400,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 4 | 4_Multi_Vlan_401 |  | Multi CoS |  | Vlan+p-bit (401,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | No Manipulation |
|  |  |  |  |  | Vlan+p-bit (401,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  |  | Vlan+p-bit (401,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 5 | 5_Multi_Vlan_500 |  | Multi CoS |  | Vlan+p-bit (500,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Pop |
|  |  |  |  |  | Vlan+p-bit (501,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  |  | Vlan+p-bit (502,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 6 | 6_Multi_Vlan_600 |  | Multi CoS |  | Vlan+p-bit (600,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Keep |
|  |  |  |  |  | Vlan+p-bit (601,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  |  | Vlan+p-bit (602,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 7 | 7_Multi_Vlan_503 |  | Multi CoS |  | Vlan+p-bit (503,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Pop |
|  |  |  |  |  | Vlan+p-bit (504,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  |  | Vlan+p-bit (505,0-1,3-4,6-7) | Best Effort | EIR=10 Mb |  |  |  |

Define the service catalog that will determine the VLAN manipulation and the OAM parameters for  the E-Line services between Aggregation devices (ETX-5) according to the following table:

| # | Service Catalog Name | Service Type | Classification | CoS | BW |  | OAM & PM | Service Vlan Manipulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Policer | Shaper |  |  |
| 8 | 8_Single_Vlan_180 | Single CoS | Vlan+p-bit (180,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Pop |
| 9 | 9_Single_Vlan_340 | Single CoS | Vlan+p-bit (340,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Swap |
| 10 | 10_Multi_Vlan_640 | Multi CoS | Vlan+p-bit (640,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Keep |
|  |  |  | Vlan+p-bit (641,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  | Vlan+p-bit (642,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
|  |  |  | Vlan+p-bit (740,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |

Define the service catalog that will determine the VLAN manipulation and the OAM parameters for each of the E-LAN services according to the following table:

| # | Service Name | Service Type | Classification | CoS | BW (Policer) |  | OAM & PM |  | Vlan Manipulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Concentrator | Non Concentrator | Concentrator to All End Points | Full Mesh |  |
| 11 | 11_Single_Vlan_800 | Single CoS | Vlan 800 | Best Effort | CIR=20M EIR=0M | CIR=10M EIR=0M | Yes | N/A | Swap Classification VLAN to S.Vlan |
| 12 | 12_Single_Vlan_801 | Single CoS | Vlan 801 | Best Effort | CIR=20M EIR=0M | CIR=10M EIR=0M | N/A | Yes | Add Service VLAN |

Define the service catalog that will determine the VLAN manipulation and the OAM parameters for  the E-Line services between Local Carriers according to the following table:

| # | Service Catalog Name | Service Type |  | Classification | CoS | BW |  | OAM & PM | Service Vlan Manipulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Policer | Shaper |  |  |
| 13 | 13_Single_Vlan_130 |  | Single CoS | Vlan+p-bit (130,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Pop |
| 14 | 14_Single_Vlan_150 |  | Single CoS | Vlan+p-bit (150,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Pop |
| 15 | 15_Single_Vlan_200 |  | Single CoS | Vlan+p-bit (200,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | No Manipulation |
| 16 | 16_Single_Vlan_330 |  | Single CoS | Vlan+p-bit (330,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Swap |
| 17 | 17_Single_Vlan_360 |  | Single CoS | Vlan+p-bit (360,0-7) | Best Effort | CIR=10M EIR=0M | N/A | Yes | Add&Swap |
| 18 | 18_Multi_Vlan_420 |  | Multi CoS | Vlan+p-bit (420,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | No Manipulation |
|  |  |  |  | Vlan+p-bit (420,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  | Vlan+p-bit (420,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 19 | 19_Multi_Vlan_460 |  | Multi CoS | Vlan+p-bit (460,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | No Manipulation |
|  |  |  |  | Vlan+p-bit (460,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  | Vlan+p-bit (460,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 20 | 20_Multi_Vlan_52x |  | Multi CoS | Vlan+p-bit (520,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Pop |
|  |  |  |  | Vlan+p-bit (521,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  | Vlan+p-bit (522,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 21 | 21_Multi_Vlan_68x |  | Multi CoS | Vlan+p-bit (680,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Keep |
|  |  |  |  | Vlan+p-bit (681,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  | Vlan+p-bit (682,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |
| 22 | 22_Multi_Vlan_66x |  | Multi CoS | Vlan+p-bit (660,5) | Platinum | CIR=9M EIR=0M | Shaper=12M | Yes | Add&Keep |
|  |  |  |  | Vlan+p-bit (661,2) | Gold | CIR=1M EIR=9M |  |  |  |
|  |  |  |  | Vlan+p-bit (662,0-1,3-4,6-7) | Best Effort | EIR=10M |  |  |  |

### Define the Test Catalog

Create the Y.1564 Test Catalog that will define the parameters that are required to test each of the services according to its SLA definitions, as following:

Test Scope: Configuration + Performance

CIR Steps(%): 25, 50, 75, 100

Test Duration Per CoS: 60 sec.

Performance Test Duration: Custom, 1 min.

No Policer on responder

## Add the entities (ETXs and Cloud)

- Create a new Level for this PoC and add the ETXs to the RADview map.

Verify the color of each entity is green or orange and that there SNMP Ping reply to each of the ETXs.

| - NE Name | - Product | - IP Address (As defined under the Preliminary Setting) |
| --- | --- | --- |
| - ETX5_A | - ETX-5300 | - 172.18.170.177 |
| - ETX5_B | - ETX-5300 | - 172.18.170.178 |
| - ETX5_C | - ETX-5300 | - 172.18.170.179 |
| - ETX-205A Tokyo | - ETX-205A | - 172.18.170.184 |
| - ETX-203AX New York | - ETX-203AX | - 172.18.170.182 |
| - ETX-220A London | - ETX-220A | - 172.18.170.183 |

Create the Clouds according to the following parameters:

| Cloud Properties |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Cloud Access Point | Connected NE | Domain Name | CoS to p-bit |
| Local Carrier_A | Local Carrier_A_Tokyo | Tokyo (Port 1) | Local_Carrier_A | Network Default |
|  | Local Carrier_A_ETX-5_A | ETX-5_A (Port 1/1) |  |  |
| Local Carrier_B | Local Carrier_B_New York | New York (Port 1) | Local_Carrier_B | Network Default |
|  | Local Carrier_B_ETX-5_B | ETX-5_B (Port 1/1) |  |  |
| Local Carrier_C | Local Carrier_C_London | London (Port 2/3) | Local_Carrier_C | Network Default |
|  | Local Carrier_C_ETX-5_C | ETX-5_C (Port 1/1) |  |  |
| International_Backbone_VPLS_MPLS | International_Backbone_VPLS_MPLS_A | ETX-5_A | International _Backbone_ VPLS_MPLS | Network Default |
|  | International_Backbone_VPLS_MPLS_B | ETX-5_B |  |  |
|  | International_Backbone_VPLS_MPLS_C | ETX-5_C |  |  |

## Create Links between the Devices

Create Links between each two entities on the map according to the following table:

|  | Links Properties |  |  |  |
| --- | --- | --- | --- | --- |
|  | Link Name |  | End Point A | End Point Z |
| Tokyo | 1 | Tokyo_ETX-2_to_Local_Cloud | ETX-205 (Port 1) | Local Carrier_A Cloud (AP Local Carrier_A_Tokyo) |
|  | 2 | Tokyo_Local_Cloud_to_ETX-5 | Local Carrier_A Cloud (AP Local Carrier_A_ETX-5_A) | ETX-5_A (Port 1/1) |
|  | 3 | Tokyo_ETX-5_to_International_Cloud | ETX-5_A (Port MC-a/1) | Internation Cloud (AP International_Backbone_VPLS_MPLS_A) |
| New York | 4 | New_York_ETX-2_to_Local_Cloud | ETX-203AX (Port 1) | Local Carrier_B Cloud (AP Local Carrier_B_New York) |
|  | 5 | New_York_Local_Cloud_to_ETX-5 | Local Carrier_B Cloud (AP Local Carrier_B_ETX-5_B) | ETX-5_B (Port 1/1) |
|  | 6 | New_York_ETX-5_to_International_Cloud | ETX-5_B(Port MC-a/1) | Internation Cloud (AP International_Backbone_VPLS_MPLS_B) |
| London | 7 | London_ETX-2_to_Local_Cloud | ETX-220A (Port 2/3) | Local Carrier_C Cloud (AP Local Carrier_C_London) |
|  | 8 | London_Local_Cloud_to_ETX-5 | Local Carrier_C Cloud (AP Local Carrier_C_ETX-5_C) | ETX-5_C (Port 1/1) |
|  | 9 | London_ETX-5_to_International_Cloud | ETX-5_B(LAG 1) | Internation Cloud (AP International_Backbone_VPLS_MPLS_C) |

## Define the Tunnels

Create tunnels  between each ETX-5 and the International Backbone Cloud according to the following table:

| Tunnel Name | Tunnel A-Point | Tunnel Z-Point | Tunnel VLAN | Tunnel Z-EP Vlan Manipulation |
| --- | --- | --- | --- | --- |
| TUNNEL_ETX5_A_CLOUD_1000 | ETX-5_A | International_Backbone_VPLS_MPLS_A | 1000 | Pop |
| TUNNEL_ETX5_A_CLOUD_2000 |  |  | 2000 | Pop |
| TUNNEL_ETX5_A_CLOUD_3000 |  |  | 3000 | Pop |
| TUNNEL_ETX5_B_CLOUD_1000 | ETX-5_B | International_Backbone_VPLS_MPLS_B | 1000 | Pop |
| TUNNEL_ETX5_B_CLOUD_2000 |  |  | 2000 | Pop |
| TUNNEL_ETX5_B_CLOUD_3000 |  |  | 3000 | Pop |
| TUNNEL_ETX5_C_CLOUD_1000 | ETX-5_C | International_Backbone_VPLS_MPLS_C | 1000 | Pop |
| TUNNEL_ETX5_C_CLOUD_2000 |  |  | 2000 | Pop |
| TUNNEL_ETX5_C_CLOUD_3000 |  |  | 3000 | Pop |

## Define the Trap Sync and Alarm Sync

- For each ETX, verify that Trap Sync Group present the IP address of the MNG station (right click on each ETX icon>Fault>Trap Sync Groups)

For each ETX, activate the Alarm Sync (right click on each ETX icon>Fault> Alarm Sync) and verify icon color updated properly according to the following table:

## Define Statistics Collection Job

The Performance Monitoring portal uses the statistics that are generated by the devices belonging to the service. A statistics collection job defines a process that gathers the statistics for the PM portal.

- Verify there is an active TFTP server installed on the PC running the RADview server application.

Create a new job to collect statistics every 15 minutes from all ETXs.

Verify that the job was successfully executed by running an Active Job to collect statistics.

After few minutes, the job will be completed with a warning. Verify that the reason for this warning is that a given NE doesn’t have statistics data to collect.

# Service Provisioning and Testing

## Backbone – E-Line over Tunnel Services between Edge Endpoints

### Test Case 1: Single CoS; New York to Tokyo; Add&Pop

Service Provisioning

- Create a Single CoS service between ETX-203AX (New York) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 1_Single_Vlan_100 | ETX-203AX Port 3 | ETX-205A Port 3 | Add&Pop | 110 | TUNNEL_ETX5_B_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with VLAN 100 p-bits 0-7 between ETX-203AX Port 3 and ETX-205A Port 3. |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 100 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio are according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 2: Single CoS; London to Tokyo; Add&Pop

##### Service Provisioning

1.	Create a Single CoS service between ETX-220A (London) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 2_Single_Vlan_300 | ETX-220A Port 2/4 | ETX-205A Port 3 | Add&Pop | 310 | TUNNEL_ETX5_C_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 300 p-bits 0-7 between ETX-220A Port 2/4 and ETX-205A Port 3. |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 300 and the same p-bit it generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 3: Multi CoS; New York to Tokyo; No Manipulation

Service Provisioning

1.	Create a Multi CoS service between ETX-203AX (New York) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 3_Multi_Vlan_400 | ETX-203AX Port 3 | ETX-205A Port 3 | No Manipulation | N/A | TUNNEL_ETX5_B_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 400 p-bit 5 - 100 Mb data stream with Vlan 400 p-bit 2 - 100 Mb data stream with Vlan 400 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 400 p-bit 5) is 9 Mb - The data rate of the Gold stream is 3M (Vlan 400 p-bit 2). - The data rate of the Best Effort stream (Vlan 400 p-bit 0,1,3,4,6,7) is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 400 p-bit 5 - 100 Mb data stream with Vlan 400 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12M - The data rate of the Platinum stream (Vlan 400 p-bit 5) is 9 Mb - The data rate of the Best Effort stream (Vlan 400 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 400 p-bit 2 - 100 Mb data stream with Vlan 400 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12M - The data rate of the Gold stream (Vlan 400 p-bit 2) is 10 Mb - The data rate of the Best Effort stream (Vlan 400 p-bit 0,1,3,4,6,7) is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 400 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12M - The data rate of the Best Effort stream (Vlan 400 p-bit 0,1,3,4,6,7) is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 4: Multi CoS; London to Tokyo; No Manipulation

Service Provisioning

1.	Create a Multi CoS service between ETX-220A (London) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 4_Multi_Vlan_401 | ETX-220A Port 2/4 | ETX-205A Port 3 | No Manipulation | N/A | TUNNEL_ETX5_C_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 401 p-bit 5 - 100 Mb data stream with Vlan 401 p-bit 2 - 100 Mb data stream with Vlan 401 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12M - The data rate of the Platinum stream (Vlan 401 p-bit 5) is 9 Mb - The data rate of the Gold stream (Vlan 401 p-bit 2) is 3 Mb - The data rate of the Best Effort stream (Vlan 401 p-bit 0,1,3,4,6,7) is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 401 p-bit 5 - 100 Mb data stream with Vlan 401 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 401 p-bit 5) is 9 Mb - The data rate of the Best Effort stream (Vlan 401 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 401 p-bit 2 - 100 Mb data stream with Vlan 401 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream (Vlan 401 p-bit 2) is 10 Mb - The data rate of the Best Effort stream (Vlan 401 p-bit 0,1,3,4,6,7) is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 401 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream (Vlan 401 p-bit 0,1,3,4,6,7) is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 5: Multi CoS; New York to Tokyo; Add&Pop

Service Provisioning

1.	Create a Multi CoS service between ETX-220A (London) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 5_Multi_Vlan_500 | ETX-203AX Port 3 | ETX-205A Port 3 | Add&Pop | 510 | TUNNEL_ETX5_B_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 500 p-bit 5 - 100 Mb data stream with Vlan 501 p-bit 2 - 100 Mb data stream with Vlan 502 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 500 p-bit 5) is 9 Mb - The data rate of the Gold stream (Vlan 501 p-bit 2) is 3 Mb - The data rate of the Best Effort stream is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 501 p-bit 5 - 100 Mb data stream with Vlan 501 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 501 p-bit 5) is 9 Mb - The data rate of the Best Effort stream (Vlan 501 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 501 p-bit 2 - 100 Mb data stream with Vlan 501 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream (Vlan 501 p-bit 2) is 10 Mb - The data rate of the Best Effort stream (Vlan 501 p-bit 0,1,3,4,6,7) is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 501 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream (Vlan 501 p-bit 0,1,3,4,6,7) is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 6: Multi CoS; New York to Tokyo; Add&Keep

Service Provisioning

1.	Create a Multi CoS service between ETX-220A (London) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 6_Multi_Vlan_600 | ETX-203AX Port 3 | ETX-205A Port 3 | Add&Keep | 610 | TUNNEL_ETX5_B_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 600 p-bit 5 - 100 Mb data stream with Vlan 601 p-bit 2 - 100 Mb data stream with Vlan 602 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb and it is tagged with S-Vlan 610 p-bit 5 and C-Vlan 600 p-bit 5. - The data rate of the Gold is 3 Mb and it is tagged with S-Vlan 610 p-bit 2 and C-Vlan 601 p-bit 2. - The data rate of the Best Effort stream is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 600 p-bit 5 - 100 Mb data stream with Vlan 602 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (S-Vlan 610 p-bit 1 and C-Vlan Vlan 602 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 600 p-bit 2 - 100 Mb data stream with Vlan 602 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 600 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 7: Multi CoS; London to Tokyo; Add&Pop

Service Provisioning

1.	Create a Multi -CoS service between ETX-220A (London) and ETX-205A (Tokyo) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 7_Multi_Vlan_503 | ETX-220A Port 2/4 | ETX-205A Port 3 | Add&Pop | 513 | TUNNEL_ETX5_C_CLOUD_2000 | TUNNEL_ETX5_A_CLOUD_3000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 503 p-bit 5 - 100 Mb data stream with Vlan 504 p-bit 2 - 100 Mb data stream with Vlan 505 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 503 p-bit 5) is 9 Mb - The data rate of the Gold stream (Vlan 504 p-bit 2) is 3 Mb - The data rate of the Best Effort stream (Vlan 505 p-bit 0,1,3,4,6,7) is 0 | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 503 p-bit 5 - 100 Mb data stream with Vlan 505 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (Vlan 505 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 504 p-bit 2 - 100 Mb data stream with Vlan 505 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 505 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

## Backbone –E-Line over Tunnel Services between Aggregation Devices

### Test Case 8: Single CoS; ETX-5_C to ETX-5_A; Add&Pop

Service Provisioning

1.	Create a Single CoS service between ETX-5_C and ETX-5_A according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 8_Single_Vlan_180 | ETX-5_C Port 1/11 | ETX-5_A Port 1/11 | Add&Pop | 1180 | TUNNEL_ETX5_C_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 180 p-bits 0-7 between ETX-5_C Port 1/11 and ETX-5_A Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 180 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
| # | Action | Expected Result | Pass/Fail |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
| # | Action | Expected Result | Pass/Fail |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
| # | Action | Expected Result | Pass/Fail |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio are according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 9: Single CoS; ETX-5_A to ETX-5_B; Add&Swap

Service Provisioning

1.	Create a Single CoS service between ETX-5_A and ETX-5_B according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Swap C.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |  | A-Point | Z-Point |
| 9_Single_Vlan_340 | ETX-5_A Port 1/11 | ETX-5_B Port 1/11 | Add&Swap | 1180 | 341 | TUNNEL_ETX5_C_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 340 p-bits 0-7 between ETX-5_A Port 1/11 and ETX-5_B Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 341 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
| # | Action | Expected Result | Pass/Fail |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
| # | Action | Expected Result | Pass/Fail |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
| # | Action | Expected Result | Pass/Fail |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio are according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 10: Multi CoS; ETX-5_B to ETX-5_C; Add&Keep

Service Provisioning

1.	Create a Multi CoS service between ETX-5_B and ETX-5_C according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Tunnel – Alternative Path |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  | A-Point | Z-Point |
| 10_Multi_Vlan_640 | ETX-5_B Port 1/11 | ETX-5_C Port 1/11 | Add&Keep | 1640 | TUNNEL_ETX5_B_CLOUD_1000 | TUNNEL_ETX5_A_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 640 p-bit 5 - 100 Mb data stream with Vlan 641 p-bit 2 - 100 Mb data stream with Vlan 642 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb and it is tagged with S-Vlan 1640 p-bit 5 and C-Vlan 640 p-bit 5. - The data rate of the Gold is 3 Mb and it is tagged with S-Vlan 1640 p-bit 2 and C-Vlan 641 p-bit 2. - The data rate of the Best Effort stream is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 640 p-bit 5 - 100 Mb data stream with Vlan 642 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (S-Vlan 1640 p-bit 1 and C-Vlan 642 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 641 p-bit 2 - 100 Mb data stream with Vlan 642 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 642 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

## Backbone – E-LAN over Tunnel services

### Test Case 11: E-LAN; Single CoS; Tokyo to New York and to London; Swap C.VLAN

Service Provisioning

1.	Create an E-LAN Single CoS service between ETX-205A (Concentrator) to and ETX-203AX (Non-Concentrator) and ETX-220A (Non-Concentrator) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  |  | OAM & PM |  | Vlan Manipulation | Swap S.Vlan |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Concentrator | Non-Concentrator | Non-Concentrator | Concentrator to all End Points | Full Mesh |  |  |
| 11_Single_Vlan_800 | ETX-205A Port 3 | ETX-203AX Port 3 | ETX-220A Port 2/4 | Yes | N/A | Swap C.Vlan to S.Vlan | 899 |

| Service Name (Service Catalog Name) | Tunnel – Alternative Path |  |  |
| --- | --- | --- | --- |
|  | C-Point | N-Point | N-Point |
| 11_Single_Vlan_800 | TUNNEL_ETX5_A_CLOUD_1000 | TUNNEL_ETX5_B_CLOUD_1000 | TUNNEL_ETX5_C_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 800 from the Concentrator towards each of the Non-Concentrators End Points |  |  |
|  | Analyze the traffic receive at the ETH generator on both Non-Concentrators sides | - The traffic rate on each Non-Concentrator side is 10 Mb and it is tagged with Vlan 899. | Pass |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 899 from both Non-Concentrator towards Concentrators End Point |  |  |
|  | Analyze the traffic receive at the ETH generator on the Concentrator side | - The total traffic rate on the Concentrator side is 20 Mb and it is tagged with Vlan 800. | Pass |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 899 between the two Non-Concentrator End Point |  |  |
|  | Analyze the traffic receive at the ETH generator on both Non-Concentrator sides | - The traffic rate on each Non-Concentrator side is 10 Mb and it is tagged with Vlan 899. |  |
| OAM Test |  |  |  |
|  | Check the OAM MEP status on the Concentrator side | - There are two MEPs configured towards the two Non-Concentrator units. - The status of each remote MEP is OK and the MAC address is indeed of the remote ETX. | Pass |
|  | Check the OAM MEP status on each Non-Concentrator side | - Each Non-Concentrator has one MEP towards the Concentrator - The status of the remote MEP is OK and the MAC address is indeed of the Concentrator ETX. |  |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb for each of the Non-Concentrators side and 20 Mb for the Concentrator side. | Pass |

### Test Case 12: E-LAN; Single CoS; Tokyo to New York and to London; Add Service VLAN

Service Provisioning

1.	Create an E-LAN Single CoS service between ETX-205A (Concentrator) to and ETX-203AX (Non-Concentrator) and ETX-220A (Non-Concentrator) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  |  | OAM & PM |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Concentrator | Non-Concentrator | Non-Concentrator | Concentrator to all End Points | Full Mesh |  |  |
| 12_Single_Vlan_801 | ETX-205A Port 3 | ETX-203AX Port 3 | ETX-220A Port 2/4 | N/A | Yes | Push | 4001 |

| Service Name (Service Catalog Name) | Tunnel – Alternative Path |  |  |
| --- | --- | --- | --- |
|  | C-Point | N-Point | N-Point |
| 12_Single_Vlan_801 | TUNNEL_ETX5_A_CLOUD_3000 | TUNNEL_ETX5_B_CLOUD_2000 | TUNNEL_ETX5_C_CLOUD_1000 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 801 from the Concentrator towards each of the Non-Concentrators End Points |  |  |
|  | Analyze the traffic receive at the ETH generator on both Non-Concentrators sides | - The traffic rate on each Non-Concentrator side is 10 Mb and it is tagged with Vlan 801. | Pass |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 801 from both Non-Concentrator towards Concentrators End Point |  |  |
|  | Analyze the traffic receive at the ETH generator on the Concentrator side | - The total traffic rate on the Concentrator side is 20 Mb and it is tagged with Vlan 801. | Pass |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 801 between the two Non-Concentrator End Point |  |  |
|  | Analyze the traffic receive at the ETH generator on both Non-Concentrator sides | - The traffic rate on each Non-Concentrator side is 10 Mb and it is tagged with Vlan 801. |  |
| OAM Test |  |  |  |
|  | Check the OAM MEP status on the Concentrator side | - There are two MEPs configured towards the two Non-Concentrator units. - The status of each remote MEP is OK and the MAC address is indeed of the remote ETX. | Pass |
|  | Check the OAM MEP status on each Non-Concentrator side | - There are two MEPs configured, one towards the Concentrator and one towards the other Non-Concentrator unit. - The status of each remote MEP is OK and the MAC address is indeed of the remote ETX. |  |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb for each of the Non-Concentrators side and 20 Mb for the Concentrator side. | Pass |

## Local Carrier – E-Line services

### Test Case 13; Single CoS; Tokyo to ETX-5_A; Add&Pop

Service Provisioning

1.	Create a Single CoS service between ETX-205A (Tokyo) and ETX-5_A according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 13_Single_Vlan_130 | ETX-205A Port 3 | ETX-5_A Port 1/11 | Add&Pop | 1130 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 130 p-bits 0-7 between ETX-205A Port 3 and ETX-5_A Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 130 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 14; Single CoS; London to ETX-5_C; Add&Pop

Service Provisioning

1.	Create a Single CoS service between ETX-220A (London) and ETX-5_C according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 14_Single_Vlan_150 | ETX-220A Port 2/4 | ETX-5_C Port 1/11 | Add&Pop | 1150 |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 150 p-bits 0-7 between ETX-220A Port 2/4 and ETX-5_C Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 150 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 15; Single CoS; New York to ETX-5_B; No Manipulation

Service Provisioning

1.	Create a Single CoS service between ETX-203AX (New York) and ETX-5_B according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 15_Single_Vlan_200 | ETX-203AX Port 3 | ETX-5_B Port 1/11 | No Manipulation | N/A |

2.	Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 200 p-bits 0-7 between ETX-203AX Port 3 and ETX-5_B Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 200 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 16; Single CoS; Tokyo to ETX-5_A; Add&Swap

Service Provisioning

- Create a Single CoS service between ETX-205A (Tokyo) and ETX-5_A according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Swap C.Vlan |
| --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |  |
| 16_Single_Vlan_330 | ETX-205A Port 3 | ETX-5_A Port 1/11 | Add&Swap | 1320 | 321 |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 330 p-bits 0-7 between ETX-205A Port 3 and ETX-5_A Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 361 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 17; Single CoS; New York to ETX-5_B; Add&Swap

Service Provisioning

- Create a Single CoS service between ETX-203AX (New York) and ETX-5_B according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan | Swap C.Vlan |
| --- | --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |  |
| 17_Single_Vlan_360 | ETX-203AX Port 3 | ETX-5_B Port 1/11 | Add&Swap | 1360 | 361 |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit 100 Mb data stream with Vlan 360 p-bits 0-7 between ETX-203AX Port 3 and ETX-5_B Port 1/11 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 10 Mb - Each data stream is tagged with Vlan 361 and the same p-bit it was generated. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 10 Mb | Pass |

### Test Case 18; Multi CoS; Tokyo to ETX-5_A; No Manipulation

Service Provisioning

- Create a Multi CoS service between ETX-205A (Tokyo) and ETX-5_A according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 18_Multi_Vlan_420 | ETX-205A Port 3 | ETX-5300_A Port 1/11 | No Manipulation | N/A |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 420 p-bit 5 - 100 Mb data stream with Vlan 420 p-bit 2 - 100 Mb data stream with Vlan 420 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 420 p-bit 5) is 9 Mb - The data rate of the Gold stream (Vlan 420 p-bit 2) is 3 Mb - The data rate of the Best Effort stream (Vlan 420 p-bit 0,1,3,4,6,7) is 0 | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 420 p-bit 5 - 100 Mb data stream with Vlan 420 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (C-Vlan 420 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 420 p-bit 2 - 100 Mb data stream with Vlan 420 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 420 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 19; Multi CoS; London to ETX-5_C; No Manipulation

Service Provisioning

- Create a Multi CoS service between ETX-220A (London) and ETX-5_C according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 19_Multi_Vlan_460 | ETX-220A Port 2/4 | ETX-5300_C Port 1/11 | No Manipulation | N/A |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 460 p-bit 5 - 100 Mb data stream with Vlan 460 p-bit 2 - 100 Mb data stream with Vlan 460 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 460 p-bit 5) is 9 Mb - The data rate of the Gold stream (Vlan 460 p-bit 2) is 3 Mb - The data rate of the Best Effort stream (Vlan 460 p-bit 0,1,3,4,6,7) is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 460 p-bit 5 - 100 Mb data stream with Vlan 460 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (Vlan 460 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 460 p-bit 2 - 100 Mb data stream with Vlan 460 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 460 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 20; Multi CoS; Tokyo to ETX-5_A; Add&Pop

Service Provisioning

- Create a Multi CoS service between ETX-205A (Tokyo) and ETX-5300_A according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 20_Multi_Vlan_520 | ETX-205A Port 3 | ETX-5_A Port 1/11 | Add&Pop | 1520 |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 520 p-bit 5 - 100 Mb data stream with Vlan 521 p-bit 2 - 100 Mb data stream with Vlan 522 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream (Vlan 520 p-bit 5) is 9 Mb - The data rate of the Gold stream (Vlan 521 p-bit 2) is 3 Mb - The data rate of the Best Effort stream (Vlan 522 p-bit 0,1,3,4,6,7) is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 520 p-bit 5 - 100 Mb data stream with Vlan 522 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (Vlan 522 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 521 p-bit 2 - 100 Mb data stream with Vlan 522 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 522 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 21; Multi CoS; ETX-5_B to New York; Add&Keep

Service Provisioning

- Create a Multi CoS service between ETX-5_B and ETX-203AX (New York) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 21_Multi_Vlan_680 | ETX-5_B Port 1/11 | ETX-203AX Port 3 | Add&Keep | 1680 |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 680 p-bit 5 - 100 Mb data stream with Vlan 681 p-bit 2 - 100 Mb data stream with Vlan 682 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb and it is tagged with S-Vlan 1680 p-bit 5 and C-Vlan 680 p-bit 5. - The data rate of the Gold is 3 Mb and it is tagged with S-Vlan 1680 p-bit 2 and C-Vlan 681 p-bit 2. - The data rate of the Best Effort stream is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 680 p-bit 5 - 100 Mb data stream with Vlan 682 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (S-Vlan 1680 p-bit 1 and C-Vlan 682 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 681 p-bit 2 - 100 Mb data stream with Vlan 682 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 682 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

### Test Case 22; Multi CoS; London to ETX-5_C; Add&Keep

Service Provisioning

- Create a Multi CoS service between ETX-5_B and ETX-203AX (New York) according to the following table:

| Service Name (Service Catalog Name) | Service End Points |  | Vlan Manipulation | S.Vlan |
| --- | --- | --- | --- | --- |
|  | A-Point | Z-Point |  |  |
| 22_Multi_Vlan_660 | ETX-220A Port 2/4 | ETX-5_B Port 1/11 | Add&Keep | 1660 |

Verify the service was successfully created.

##### Test Procedure

| Rate & CoS Preservation Test |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 660 p-bit 5 - 100 Mb data stream with Vlan 661 p-bit 2 - 100 Mb data stream with Vlan 662 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb and it is tagged with S-Vlan 1660 p-bit 5 and C-Vlan 660 p-bit 5. - The data rate of the Gold is 3 Mb and it is tagged with S-Vlan 1660 p-bit 2 and C-Vlan 661 p-bit 2. - The data rate of the Best Effort stream is 0. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 660 p-bit 5 - 100 Mb data stream with Vlan 662 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Platinum stream is 9 Mb - The data rate of the Best Effort stream (S-Vlan 1660 p-bit 1 and C-Vlan 662 p-bit 0,1,3,4,6,7) is 3 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 661 p-bit 2 - 100 Mb data stream with Vlan 662 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Gold stream is 10 Mb - The data rate of the Best Effort stream is 2 Mb. | Pass |
|  | Set the ETH generator to transmit: - 100 Mb data stream with Vlan 662 p-bit 0,1,3,4,6,7 |  |  |
|  | Analyze the traffic receive at the ETH generator on both sides | - The total traffic rate is 12 Mb - The data rate of the Best Effort stream is 12 Mb. | Pass |
| OAM Test |  |  |  |
|  | Check the OAM MEP status | The Remote MEP status is OK and the MAC address is indeed of the remote ETX. | Pass |
| Y.1564 Test |  |  |  |
|  | Run Y.1564 test according to the test catalog you created – see 3.1.1.4 Define the Test Catalog | - Y.1564 Test was successfully completed. - All Y.1564 Test results are passed. | Pass |
|  | Open the Y.1564 Summary report | - FLR (Frame Loss Ratio), FTD (Frame Transfer Delay), FDV (Frame Delay Variation) and Availability are within SAC (Service Acceptance Criteria) limits | Pass |
| PM Portal |  |  |  |
|  | Let the traffic run for about an hour and open the PM Portal report |  |  |
|  | Check that the OAM KPI parameters value for each service (Availability, Delay , Frame Loss ratio) are according to what you configured under the Policies – 2.2.2 Define the Policies | Availability, Delay, Frame Loss ratio is according to the Policies. | Pass |
|  | Check the Utilization of the services | The utilization for the service is 12 Mb | Pass |

Appendix A - ETX-5 Preliminary Setting

ETX-5_A

##### Hardware, Queues and Port Configuration

#HW Configuration #config slot 1 card-type eth gbe-20-sfp #config slot 1 no shutdown #*when IO card ready run script #***************************Classification_Key*************************** config port ethernet main-a/1 classification-key vlan inner-vlan p-bit config port ethernet 1/1 classification-key vlan p-bit config port ethernet 1/11 classification-key vlan p-bit #***************************IO_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_2_level_default_IO1/1 inherited-from q_group_2_level_default exit all configure qos queue-group-profile q_group_2_level_default_IO1/11 inherited-from q_group_2_level_default exit all #***************************MC_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_3_level_default_MC_a/1 inherited-from q_group_3_level_default exit all #***************************SAG_QUEUE_GROUP****************************** configure qos queue-group-profile q_group_2_level_default_SAG1/1 inherited-from q_group_SAG_2_level_default exit all #***************************BIND_QG_TO_PORTS***************************** config port ethernet main-a/1 queue-group profile q_group_3_level_default_MC_a/1 config port sag 1/1 queue-group profile q_group_2_level_default_SAG1/1 config port ethernet 1/1 queue-group profile q_group_2_level_default_IO1/1 config port ethernet 1/11 queue-group profile q_group_2_level_default_IO1/11 exit all #***************************Color-Mapping******************************* config qos color-map-profile dei-color-map classification dei #******************************PORT_ACTIVATION************************** config port ethernet main-a/1 no shutdown config port ethernet 1/1 no shutdown config port ethernet 1/11 no shutdown

##### Local Management Set Up

#***************************SNMP Configuration*************************** configure system name "ETX-5_A" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #******************************configure port svi************************** exit all configure port svi 1 bridge exit all configure port svi 2 bridge exit all configure port svi 99 router exit all #***************************Marking-Mapping******************************* config qos marking-profile "mark1" color-aware green-yellow dei mapping exit all #*************************** Bridge 1 ****************************** config bridge 1 port 1 bind svi 1 no shutdown exit port 2 bind svi 2 no shutdown exit vlan 4094 tagged-egress 1,2 exit all #**************** Classifier Profile Configuration ********************** config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 exit all #************************Flows_Inband_Configuration************************* config flows flow "mng_in_bridge" classifier "VLAN_4094" ingress-port ethernet main-a/1 egress-port svi 1 no shutdown exit all config flows flow "mng_bridge_router" classifier "VLAN_4094" ingress-port svi 2 egress-port svi 99 vlan-tag pop vlan no shutdown exit all config flows flow "mng_router_bridge" classifier "all" ingress-port svi 99 egress-port svi 2 vlan-tag push vlan 4094 p-bit profile "mark1" tag-ether-type 0x8100 no shutdown exit all config flows flow "mng_out_bridge" classifier "VLAN_4094" ingress-port svi 1 egress-port ethernet main-a/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all #*************************Router _Configuration************************************ config router 1 name "Router#1" interface 1 address 172.18.170.177/24 bind svi 99 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all

##### Management towards ETX-205A (Tokyo)

#******************************configure port svi************************** exit all configure port svi 3 bridge exit all #*************************** Bridge 1 ****************************** config bridge 1 port 3 bind svi 3 no shutdown exit vlan 4094 tagged-egress 3 exit all #************************Flows_Inband_Configuration************************* config flows flow "mng_eth_1_1_to_SAP_1_1_1" classifier "VLAN_4094" ingress-port ethernet 1/1 egress-port sap 1/1/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all config flows flow "mng_SAP_1_1_1_to_bridge" classifier "VLAN_4094" ingress-port sap 1/1/1 egress-port svi 3 no shutdown exit all config flows flow "mng_bridge_to_eth_1_1" classifier "VLAN_4094" ingress-port svi 3 egress-port ethernet 1/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all

ETX-5_B

##### Hardware, Queues and Port Configuration

#*******Define IO card type first #config slot 1 card-type eth gbe-20-sfp #config slot 1 no shutdown #*when IO card ready run script #***************************Classification_Key*************************** config port ethernet main-a/1 classification-key vlan inner-vlan p-bit config port ethernet 1/1 classification-key vlan p-bit config port ethernet 1/11 classification-key vlan p-bit #***************************IO_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_2_level_default_IO1/1 inherited-from q_group_2_level_default exit all configure qos queue-group-profile q_group_2_level_default_IO1/11 inherited-from q_group_2_level_default exit all #***************************MC_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_3_level_default_MC_a/1 inherited-from q_group_3_level_default exit all #***************************SAG_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_2_level_default_SAG1/1 inherited-from q_group_SAG_2_level_default exit all #***************************BIND_QG_TO_PORTS******************************* config port ethernet main-a/1 queue-group profile q_group_3_level_default_MC_a/1 config port sag 1/1 queue-group profile q_group_2_level_default_SAG1/1 config port ethernet 1/1 queue-group profile q_group_2_level_default_IO1/1 config port ethernet 1/11 queue-group profile q_group_2_level_default_IO1/11 #***************************Color-Mapping******************************* config qos color-map-profile dei-color-map classification dei #******************************PORT_ACTIVATION************************** config port ethernet main-a/1 no shutdown config port ethernet 1/1 no shutdown config port ethernet 1/11 no shutdown

##### Local Management Set Up

#***************************SNMP Configuration*************************** configure system name "ETX-5_B" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #******************************configure port svi************************** exit all configure port svi 1 bridge exit all configure port svi 2 bridge exit all configure port svi 99 router exit all #***************************Marking-Mapping******************************* config qos marking-profile "mark1" color-aware green-yellow dei mapping exit all #*************************** Bridge 1 ****************************** config bridge 1 port 1 bind svi 1 no shutdown exit port 2 bind svi 2 no shutdown exit vlan 4094 tagged-egress 1,2 exit all #**************** Classifier Profile Configuration ********************** config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 exit all #************************Flows_Inband_Configuration************************* config flows flow "mng_in_bridge" classifier "VLAN_4094" ingress-port ethernet main-a/1 egress-port svi 1 no shutdown exit all config flows flow "mng_bridge_router" classifier "VLAN_4094" ingress-port svi 2 egress-port svi 99 vlan-tag pop vlan no shutdown exit all config flows flow "mng_router_bridge" classifier "all" ingress-port svi 99 egress-port svi 2 vlan-tag push vlan 4094 p-bit profile "mark1" tag-ether-type 0x8100 no shutdown exit all config flows flow "mng_out_bridge" classifier "VLAN_4094" ingress-port svi 1 egress-port ethernet main-a/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all #*************************Router _Configuration************************************ config router 1 name "Router#1" interface 1 address 172.18.170.178/24 bind svi 99 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all

### Management towards ETX-203AX (New York)

#******************************configure port svi************************** exit all configure port svi 3 bridge exit all #*************************** Bridge 1 ****************************** config bridge 1 port 3 bind svi 3 no shutdown exit vlan 4094 tagged-egress 3 exit all #************************Flows_Inband_Configuration************************* config flows flow "mng_eth_1_1_to_SAP_1_1_1" classifier "VLAN_4094" ingress-port ethernet 1/1 egress-port sap 1/1/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all config flows flow "mng_SAP_1_1_1_to_bridge" classifier "VLAN_4094" ingress-port sap 1/1/1 egress-port svi 3 no shutdown exit all config flows flow "mng_bridge_to_eth_1_1" classifier "VLAN_4094" ingress-port svi 3 egress-port ethernet 1/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all

ETX-5_C

##### Hardware, Queues, LAG and Port Configuration

#*******Define IO card type first #config slot 1 card-type eth gbe-20-sfp #config slot 1 no shutdown #*when IO card ready run script #***************************Classification_Key*************************** config port ethernet 1/1 classification-key vlan p-bit config port ethernet 1/11 classification-key vlan p-bit config port ethernet main-a/3 classification-key vlan p-bit config port ethernet main-a/4 classification-key vlan p-bit #***************************IO_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_2_level_default_IO1/1 inherited-from q_group_2_level_default exit all configure qos queue-group-profile q_group_2_level_default_IO1/11 inherited-from q_group_2_level_default exit all #***************************MC_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_3_level_default_MC_LAG inherited-from q_group_3_level_default exit all configure qos queue-group-profile q_group_3_level_default_MC_a/3 inherited-from q_group_3_level_default exit all configure qos queue-group-profile q_group_3_level_default_MC_a/4 inherited-from q_group_3_level_default exit all #***************************SAG_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_2_level_default_SAG1/1 inherited-from q_group_SAG_2_level_default exit all #***************************BIND_QG_TO_PORTS******************************* config port sag 1/1 queue-group profile q_group_2_level_default_SAG1/1 config port ethernet 1/1 queue-group profile q_group_2_level_default_IO1/1 config port ethernet 1/11 queue-group profile q_group_2_level_default_IO1/11 config port ethernet main-a/3 queue-group profile q_group_3_level_default_MC_a/3 config port ethernet main-a/4 queue-group profile q_group_3_level_default_MC_a/4 #***************************Color-Mapping******************************* config qos color-map-profile dei-color-map classification dei #******************************LAG_1_ACTIVATION************************** config port lag 1 admin-key ten-giga-ethernet bind ethernet main-a/1 bind ethernet main-a/2 queue-group profile q_group_3_level_default_MC_LAG lacp tx-activity active tx-speed fast classification-key vlan inner-vlan p-bit no shutdown exit all #******************************* L2CP_PROFILE ********************************* config port l2cp l2cpLAG_OAM mac 01-80-c2-00-00-02 peer exit all #***************************** Capture Lacp packets *************************** classifier-profile ClassUntagged match-any match untagged config flows flow Lacp classifier ClassUntagged ingress-port lag 1 egress-port ethernet main-a/4 queue-map-profile QueueMapDefaultProfile block 0/1 l2cp profile l2cpLAG_OAM no shutdown exit all #****************************** PORT_ACTIVATION ************************** config port ethernet main-a/1 no shutdown config port ethernet main-a/2 no shutdown config port ethernet 1/1 no shutdown config port ethernet 1/11 no shutdown

##### Local Management Set Up

#***************************SNMP Configuration*************************** configure system name "ETX-5_C" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #******************************configure port svi************************** exit all configure port svi 1 bridge exit all configure port svi 2 bridge exit all configure port svi 99 router exit all #***************************Marking-Mapping******************************* config qos marking-profile "mark1" color-aware green-yellow dei mapping #*************************** Bridge 1 ****************************** config bridge 1 port 1 bind svi 1 no shutdown exit port 2 bind svi 2 no shutdown exit vlan 4094 tagged-egress 1,2 exit all #**************** Classifier Profile Configuration ********************** config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 #************************Flows_Inband_Configuration************************* config flows flow "mng_in_bridge" classifier "VLAN_4094" ingress-port lag 1 egress-port svi 1 no shutdown exit all config flows flow "mng_bridge_router" classifier "VLAN_4094" ingress-port svi 2 egress-port svi 99 vlan-tag pop vlan no shutdown exit all config flows flow "mng_router_bridge" classifier "all" ingress-port svi 99 egress-port svi 2 vlan-tag push vlan 4094 p-bit profile "mark1" tag-ether-type 0x8100 no shutdown exit all config flows flow "mng_out_bridge" classifier "VLAN_4094" ingress-port svi 1 egress-port lag 1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all #*************************Router _Configuration************************************ config router 1 name "Router#1" interface 1 address 172.18.170.179/24 bind svi 99 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all

##### Management towards ETX-220A (London)

#******************************configure port svi************************** exit all configure port svi 3 bridge exit all #*************************** Bridge 1 ****************************** config bridge 1 port 3 bind svi 3 no shutdown exit vlan 4094 tagged-egress 3 exit all #************************Flows_Inband_Configuration************************* config flows flow "mng_eth_1_1_to_SAP_1_1_1" classifier "VLAN_4094" ingress-port ethernet 1/1 egress-port sap 1/1/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all config flows flow "mng_SAP_1_1_1_to_bridge" classifier "VLAN_4094" ingress-port sap 1/1/1 egress-port svi 3 no shutdown exit all config flows flow "mng_bridge_to_eth_1_1" classifier "VLAN_4094" ingress-port svi 3 egress-port ethernet 1/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 no shutdown exit all

Appendix B - ETX-2 Preliminary Setting

B.1 ETX-205A Tokyo

##### Queues and Management Configuration

#***************************QUEUE_GROUP******************************* exit all config qos queue-group-profile QGN1 exit all config port eth 1 queue-group profile QGN1 exit all #***************************SNMP Configuration************************ configure system name "ETX_Tokyo" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #*************************** Inband MNG *********************** config port svi 1 no shutdown exit all config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 config flows flow "Eth_1_svi_1" classifier "VLAN_4094" no policer vlan-tag pop vlan ingress-port ethernet 1 egress-port svi 1 queue 0 no shutdown exit all config flows flow "svi_1_Eth_1" classifier "all" no policer ingress-port svi 1 egress-port ethernet 1 queue 0 block 0/1 vlan-tag push vlan 4094 p-bit fixed 0 no shutdown exit all config router 1 name "Router#1" interface 1 address 172.18.170.184/24 bind svi 1 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all

B.2 ETX-203AX New York

##### Queues and Management Configuration

#***************************QUEUE_GROUP******************************* exit all config qos queue-group-profile QGN1 exit all config port eth 1 queue-group profile QGN1 exit all #***************************SNMP Configuration************************ configure system name "ETX_New_York" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #*************************** Inband MNG *********************** config port svi 1 no shutdown exit all config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 config flows flow "Eth_1_svi_1" classifier "VLAN_4094" no policer vlan-tag pop vlan ingress-port ethernet 1 egress-port svi 1 queue 0 no shutdown exit all config flows flow "svi_1_Eth_1" classifier "all" no policer ingress-port svi 1 egress-port ethernet 1 queue 0 block 0/1 vlan-tag push vlan 4094 p-bit fixed 0 no shutdown exit all config router 1 name "Router#1" interface 1 address 172.18.170.182/24 bind svi 1 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all

B.3 ETX-220A London

##### Queues and Management Configuration

#***************************QUEUE_GROUP Port 2/3 ******************************* config qos queue-group-profile QGN2_3 exit all config port eth 2/3 queue-group profile QGN2_3 exit all config qos queue-group-profile QGN2_4 exit all config port eth 2/4 queue-group profile QGN2_4 exit all #***************************SNMP Configuration******************************* configure system name "ETX_London" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #*************************** Inband MNG *************************** config port svi 1 no shutdown exit all config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 config flows flow "ETH_2_3_SVI_1" classifier "VLAN_4094" no policer vlan-tag pop vlan ingress-port ethernet 2/3 egress-port svi 1 queue 0 no shutdown exit all config flows flow "SVI_1_ETH_2_3" classifier "all" no policer ingress-port svi 1 egress-port ethernet 2/3 queue 0 block 0/1 vlan-tag push vlan 4094 p-bit fixed 0 no shutdown exit all config router 1 name "Router#1" interface 1 address 172.18.170.183/24 bind svi 1 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all