# TDM Service over Packet Networks

PoC Template #3

| Last Updated: | Jan 30, 2015 |
| --- | --- |
| Doc. Version: | 1.0 |
| Doc. Owner | Ronen Rick |

### Contents

1.	Introduction	4

1.1.	Purpose	4

1.2.	Layout Overview	4

1.2.1.	Application Diagram	4

1.2.2.	Devices under Test	5

1.2.3.	Devices Under Test	6

2.	Preliminary Configuration and Management	7

2.1.	Setting SDH	7

2.1.1.	Setting ETX-5 Aggregation POP	7

2.1.2.	ETX-205 HQ Preliminary Setting	7

2.1.3.	IPmux-24 Preliminary Setting	8

2.1.4.	LA-210 Preliminary Setting	8

2.1.5.	MiTOP Preliminary Setting	8

2.2.	Devices Preliminary Setting – SONET (OC-3)	9

2.2.1.	ETX-5 Aggregation POP Preliminary Setting	9

2.2.2.	ETX-205 HQ Preliminary Setting	9

2.2.3.	IPmux-24 Preliminary Setting	9

2.2.4.	MiTOP Preliminary Setting	10

3.	TDM Service Provisioning and Testing	11

3.1.	TDM Services STM-1 – CESoPSN, UDP/IP between Edge Endpoints	11

3.1.1.	Test Case 1: PW1-4 between Aggregation POP to IPmux-24	11

3.1.2.	Test Case 2: PW5-8 between Aggregation POP to HQ, ETX-205	12

3.1.3.	Test Case 3: PW9 between Aggregation POP and MiTOP(E1)	13

3.1.4.	Test Case 4: PW10 between Aggregation POP to LA-210	14

3.2.	TDM Services OC-3 – SAToP, MEF8 between Edge Endpoints	16

3.2.1.	Test Case 1: PW1-4 between Aggregation POP to IPmux-24	16

3.2.2.	Test Case 2: PW5-8 between Aggregation POP to HQ, ETX-205	17

3.2.3.	Test Case 3: PW9 between Aggregation POP to MiTOP (T1)	18

A.	Appendix A – STM1	20

A.1.	Aggregation POP – ETX-5 Preliminary Setting	20

A.2.	HQ – ETX-205 Preliminary Setting	23

A.3.	IPmux-24 – Management, TDM type and Mux Method Setting	25

A.4.	LA-210 – Preliminary Setting	32

A.5.	MiTOP – Preliminary Setting	34

B.	Appendix B – SONET (OC-3)	35

B.1.	Aggregation POP – ETX-5 Preliminary Setting	35

B.2.	HQ - ETX-205 Preliminary Setting	39

B.3.	IPMUX-24 – Management, TDM type and Mux Method Setting	42

B.4.	MiTOP – Preliminary Setting	49

Appendix C – IEEE 1588v2 Frequency Synchronization (	52

C.	SDH and SONET)	52

C.1.	ETX-5 Master Configuration	52

C.2.	ETX-205 Slave Configuration	53

D.	Appendix D – SDH – CESoPSN (UDP/IP) Services	55

D.1.	ETX-5  IPMUX-24 Services Configuration	55

D.2.	ETX-5  ETX-205 Services Configuration	62

D.3.	ETX-5   MiTOP Services Configuration	67

D.4.	ETX-5  LA-210 Services Configuration	72

E.	Appendix E – SONET – SATOP (MEF 8)	74

E.1.	ETX-5    IPmux-24 Services Configuration	74

E.2.	ETX-5  ETX-205 Services Configuration	81

E.3.	ETX-5   MiTOP Services Configuration	86

# Introduction

## Purpose

This document describes various testing scenarios for deployment of TDM service emulation over packet networks, using TDM pseudowire technology as implemented in selected RAD products.

This type of solution allows incumbent service providers to keep revenue flow and customer loyalty by bundling packet and TDM services on the same packet transport.  Alternative service providers are able to add leased lines services to their service portfolio offering, thereby attracting new customers.

The deployment of TDM emulation can match various network topologies and service scenarios, terminating pseudowires at the desired location – customer site-to-customer site, customer site-to-POP/network, or POP-to-POP.

TDM pseudowires can be transported over Ethernet or IP transport (MEF-8/UDP-IP encapsulation). RAD products support  the relevant industry standard (CESoPSN/SAToP) with flexible clock synchronization scenarios.

## Layout Overview

### Application Diagram

### Devices under Test

| MiTOP-E1/T1 TDM pseudowire access gateway extending TDM-based services over packet-switched networks. Housed in a Small Form-Factor Pluggable (SFP) enclosure, it is designed for quick and simple insertion into any Fast or Gigabit Ethernet port with an MSA-compatible socket. |  |
| --- | --- |
| ETX-205A/E1 High performance NTE delivering SLA-based Ethernet business services including Synchronous Ethernet, IEEE1588-2008, 2 x 1GbE ports in NNI and 4 x 1GbE port in UNI, E1/T1 pseudowire services per MEF-8, UDP/IP, in SAToP and CESoP modes and with CAS. |  |
| LA-210 Mid-band Ethernet access up to 22 Mbps using EFM bonding. Up to four pairs of EFM bonded SHDSL.bis uplink lines. Up to four Fast Ethernet user ports. Pseudowire support for E1, V.35 or X.21 traffic. MEF-9 and MEF-14 EPL and EVPL certified. Advanced QoS mechanism per EVC/EVC.CoS. Ethernet link and service OAM with performance monitoring for end-to-end SLA control. Multi-standard pseudowire support for legacy services over PSN. |  |
| IPmux-24/IPmux-216 Up to four (IPmux-24), eight or 16 (IPmux-216) E1 or T1 TDM user ports. Three SFP-based fiber or copper Fast Ethernet or Gigabit Ethernet interfaces. Multi-standard hardware-based TDM pseudowire: TDMoIP, CESoPSN, SAToP, HDLCoPSN, CESoETH. ITU-T G.8032 Ethernet Ring Protection Switching (ERPS) for sub-50 ms restoration; Ethernet link and TDM pseudowire redundancy. Ethernet OAM: IEEE 802.3-2005 (formerly 802.3ah), 802.1ag/ITU-T Y.1731 (CFM). High precision clock recovery for 2G/3G cellular traffic over PSN. QoS per 802.1p, ToS/DSCP, EXP. MEF-9, MEF-14 certified for EPL, EVPL services |  |
| ETX-5300A Aggregates up 80 FE/GE links from remote NTEs with advanced traffic management; highly accurate, hardware-based OAM and performance monitoring; 4 x 10GbE interfaces in the main module; power dissipation up to 500W; form-factor, 3U modular system. Extensive Sync-E, 1588v2support, including 1588 Grandmaster. Extensive TDM pseudowire support: CESoPSN, SAToP,CESoETH (MEF-8),UDP/IP encapsulation. |  |

### Devices Under Test

The following tables contain the elements used in this PoC testing.

##### RAD POC Proposal BOM

| Product | Ordering Option | Description | QTY | SW Version |
| --- | --- | --- | --- | --- |
| ETX-5300 | ETX-5300A/AC | 5700040000 | 1 | 2.2.0(0.22) |
|  | ETX-5300A-MC/4XFP/AT | 6170210000 | 1 |  |
|  | ETX-5300A-MS/CH/S4 | 6170120000 | 1 |  |
| ETX-205A | ETX-205A/AC/19/4E1T1 | 5270480000 | 1 | 5.7.0(0.59) |
| IPmux-24 | IPMUX-24/A/4E1/N/N/N | 4880820000 | 1 | 3.52 |
| LA-210 | LA-210/ESHDSL/4W/4ETH/E1 | 5040590000 | 1 | 3.61.08 |
| MiTOP | MITOP-E1T1/GE | 5180130000 | 1 | 3.0.0(0.19) |
| SFP | SFP-2 | 1278020000 | 2 |  |
|  | SFP-6 | 1278050000 | 2 |  |
|  | XFP-1D | 1279010000 | 1 |  |
| RADview | RADVIEW-PC/PMSM/DEMO | 5760260000 | 1 | 4.0.0(0.802) + ems40-v3.9.5 Patch |
|  | RV-LIC/ENW |  |  |  |

##### Test Equipment

| Function | Unit | Notes |
| --- | --- | --- |
| TDM Generator | 2 High Speed interfaces | SONET/STM1 |
| Fiber Cable | SM Fiber for SFP-6,SFP-2 and XFP-1D |  |
| Terminal Configuration Cable | RAD CBL-DB9-RJ45 | Supplied |

##### PSN Switch (Optional order for L2 switch)

| Product | Ordering Option | Description | QTY | SW Version |
| --- | --- | --- | --- | --- |
| ETX-1002 | ETX-1002-PS/AC | 3770860000 | 1 | 1.0.4 |

##### E1/T1 TDM loop electrical schema

| Pin 1 to Pin 4 Pin 2 to Pin 5 |  |
| --- | --- |

# Preliminary Configuration and Management

Note: RADview does not support creating TDM services. It supports a zoom of network elements. In-band management is used for this PoC.

Creating the services between the different locations will be done manually via inband. RADview is used for traps/alarms and monitors the device. However there are several parameters which need to be configured via CLI before beginning to work via inband.

For each High Speed Protocol a dedicated appendix is presented. Changes between OC-3 and STM-1 and E1 to T1 and vice versa are required and appear in the appendix.

Before configuring of the all elements, set to factory default.

## Setting SDH

### Setting ETX-5 Aggregation POP

- Access ETX-5 unit via serial (baud rate 9600) and verify that the ETX-5300A- MS/CH/S4 module appears as the actual installed module in slot 1 (Configuration> show config cards-summary)

- Configure ETX-5 according to the scripts in Appendix A.1:

Factory default

Hardware, Queues and Port Configuration scripts.

- After running the scripts, verify the provisioned I/O modules are updated properly (Configuration> show config cards-summary).

Local Management Set Up configuration script.

- Configure STM1 (High Level) and Protection configuration script.

- Connect STM1-TTC and Check that High Level is synchronized.

- Verify ping replies from the Management station towards of the ETX-5

- Save the configuration of the ETX-5 device.

### ETX-205 HQ Preliminary Setting

Access ETX-205 unit via serial (baud rate 9600)

Configure ETX-205 according to the scripts in Appendix A.2:

Factory default

Queues Configuration

Local Management Set Up configuration script.

Verify ping replies from the Management station towards of the ETX-205.

Save the configuration of the ETX-205 device.

### IPmux-24 Preliminary Setting

- Access IPmux-24 unit via serial (baud rate 115200)

Configure Impux-24 according to the screen shots in Appendix A.3:

Factory default

TDM Type Setting – for E1, if device is different.

Mux method – Destination Port is standard

Management Configuration

Verify ping replies from the Management station towards of the IPmux-24.

Save the configuration of the IPmux-24 device when required.

### LA-210 Preliminary Setting

- Access LA-210 unit via serial (baud rate 9600)

Configure LA-210 according to the scripts in Appendix A.4:

Factory default

TDM Type Setting – for E1, if device is different.

Mux method – Destination Port is standard

Management Configuration

Verify ping replies from the Management station towards of the LA-210.

Save the configuration of the LA-210 device.

### MiTOP Preliminary Setting

MiTOP can be approached via a standard L2 switch or using SFP-CA (Appendix A.5).

For both options:

Configure a PC network adapter with IP 192.168.205.10 mask 255.255.255.0

For PC

- Install MiTOP in a standard L2 switch

Connect ETH cable from PC s network adapter to any port on the L2 switch

For SFP-CA

Install MiTOP in SFP-CA adapter

Connect the SFP-CA adapter to a USB on PC

Configure MiTOP according to the screen shots in Appendix A.5:

Factory default – use dip switch roll.

Use Microsoft Explorer or Google Chrome and navigate to http://192.168.205.1

Configure the host IP parameters.

Verify ping replies from the Management station towards of the LA-210.

Save the configuration of the MiTOP device when required.

## Devices Preliminary Setting – SONET (OC-3)

### ETX-5 Aggregation POP Preliminary Setting

- Access ETX-5 unit via serial (baud rate 9600)and verify that the ETX-5300A- MS/CH/S4 module appears as the actual installed module in slot 1 (Configuration> show config cards-summary)

Configure ETX-5 according to the scripts in Appendix B.1:

Factory default

Hardware, Queues and Port Configuration scripts.

After running the scripts, verify the provisioned I/O modules are updated properly (Configuration> show config cards-summary).

Local Management Set Up configuration script.

Configure SONET (High Level) and Protection configuration script.

Connect OC3-TTC and Check that High Level is synchronized.

MAC address – write down for remote device configuration.

Verify ping replies from the Management station towards of the ETX-5.

Save the configuration of the ETX-5 device.

### ETX-205 HQ Preliminary Setting

- Access ETX-205 unit via serial (baud rate 9600)

Configure ETX-205 according to the scripts in Appendix B.2:

Factory default

Queues Configuration

Local Management Set Up configuration script.

TDM Type setting - change TDM frame type to DS1

MAC address – write down for remote device configuration.

Verify ping replies from the Management station towards of the ETX-205.

Save the configuration of the ETX-205 device.

### IPmux-24 Preliminary Setting

- Access IPmux-24 unit via serial (baud rate 115200)

Configure Impux-24 according to the screen shots in Appendix B.3:

Factory default

TDM Type setting - change TDM frame type to DS1

Mux method – Destination Port is standard

Management Configuration

MAC address – write down for remote device configuration

Verify ping replies from the Management station towards of the IPmux-24.

Save the configuration of the IPmux-24 device when required.

### MiTOP Preliminary Setting

MiTOP can be approached via a standard L2 switch or use SFP-CA (Appendix A.5)

For both options:

Configure a PC network adapter with IP 192.168.205.10 mask 255.255.255.0

For PC

- Install MiTOP in a standard L2 switch

Connect ETH cable from PC s network adapter to any port on the L2 switch

For SFP-CA

Install MiTOP in SFP-CA adapter

Connect the SFP-CA adapter to a USB on PC

Configure MiTOP according to the screen shots in Appendix A.5:

Factory default – use dip switch roll.

Use Microsoft Explorer or Google Chrome and navigate to http://192.168.205.1

Configure the host IP parameters.

TDM Type setting - change TDM frame type to DS1

MAC address – write down for remote device configuration

Verify ping replies from the Management station towards of the LA-210.

Save the configuration of the MiTOP device when required.

# TDM Service Provisioning and Testing

## TDM Services STM-1 – CESoPSN, UDP/IP between Edge Endpoints

### Test Case 1: PW1-4 between Aggregation POP to IPmux-24

Service Provisioning

- Configure ETX-5 Services PW1-PW4 according to scripts in Appendix D.1

Configure IPMUX-24 Services PW1-PW4 according to scripts in Appendix D.1

Connect STM1-TTC to TDM port 1 on the ETX-5 MS

Connect E1 TDM Loop to each of the TDM port in the IPMUX-24

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit E1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and IPMUX-24 | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and IPMUX-24 | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 1 | 1 | 1 | 1 |
|  | PW 2 | 2 | 1 | 1 |
|  | PW 3 | 3 | 1 | 1 |
|  | PW 4 | 1 | 2 | 1 |

###### Protection testing

- Connect E1-TTC to IPMUX-24 TDM port with Protection time measurement feature.

- Connect STM1 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect STM port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect STM port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

Note: Perform each test per TDM port of IPmux-24

### Test Case 2: PW5-8 between Aggregation POP to HQ, ETX-205

Service Provisioning

- Configure ETX-5 Services PW5-PW8 according to scripts in Appendix  D.2

Configure ETX-205 Services PW5-PW8 according to scripts in Appendix  D.2

Configure ETX-5 Master configuration according to script Appendix C.1

Configure ETX-205 Slave configuration according to script Appendix C.2

Connect STM1-TTC to TDM port 1 on the ETX-5 MS

Connect E1 TDM Loop to each of the TDM port in the ETX-205

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit E1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and ETX-205 | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and ETX-205 | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 5 | 2 | 2 | 1 |
|  | PW 6 | 3 | 2 | 1 |
|  | PW 7 | 1 | 3 | 1 |
|  | PW 8 | 2 | 3 | 1 |

###### Protection testing

- Connect E1-TTC to ETX-205 TDM port with Protection time measurement feature.

Connect STM1 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect STM port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect STM port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

Note: Perform each test per TDM port of ETX-205

### Test Case 3: PW9 between Aggregation POP and MiTOP(E1)

Service Provisioning

- Configure ETX-5 Services PW9 according to scripts in Appendix D.3

- Configure MiTOP Services PW9 according to scripts in Appendix D.3

- Connect STM1-TTC to TDM port 1 on the ETX-5 MS

- Connect E1 TDM Loop to the TDM port in the MiTOP

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit E1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and MiTOP | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and MiTOP | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 9 | 3 | 3 | 1 |

###### Protection testing

- Connect E1-TTC to MiTOP TDM port with Protection time measurement feature.

- Connect STM1 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect STM port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect STM port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

### Test Case 4: PW10 between Aggregation POP to LA-210

Service Provisioning

- Configure ETX-5 Services PW10 according to scripts in Appendix  D.4

Configure LA-210 Services PW10 according to scripts in Appendix  D.4

Connect STM1-TTC to TDM port 1 on the ETX-5 MS

Connect E1 TDM Loop to the TDM port in the LA-210

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit E1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and LA-210 | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and LA-210 | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 10 | 1 | 4 | 1 |

###### Protection testing

Connect E1-TTC to LA-210 TDM port with Protection time measurement feature.

Connect STM1 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect STM port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect STM port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

## TDM Services OC-3 – SAToP, MEF8 between Edge Endpoints

### Test Case 1: PW1-4 between Aggregation POP to IPmux-24

Service Provisioning

- Configure ETX-5 Services PW1-PW4 according to scripts in Appendix  E.1

Configure IPMUX-24 Services PW1-PW4 according to scripts in Appendix  E.1

Connect OC3-TTC to TDM port 1 on the ETX-5 MS

Connect T1 TDM Loop to each of the TDM port in the IPMUX-24

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit T1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and IPMUX-24 | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and IPMUX-24 | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 1 | 1 | 1 | 1 |
|  | PW 2 | 2 | 1 | 1 |
|  | PW 3 | 3 | 1 | 1 |
|  | PW 4 | 1 | 2 | 1 |

###### Protection testing

Connect T1-TTC to IPMUX-24 TDM port with Protection time measurement feature.

Connect OC3 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect OC3 port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect OC3 port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

Note: Perform each test per TDM port of IPmux-24

### Test Case 2: PW5-8 between Aggregation POP to HQ, ETX-205

Service Provisioning

- Configure ETX-5 Services PW5-PW8 according to scripts in Appendix  E.2

Configure ETX-205 Services PW5-PW8 according to scripts in Appendix  E.2

Configure ETX-5 Master configuration according to script C.1

Configure ETX-205 Slave configuration according to script C.2

Connect OC3-TTC to TDM port 1 on the ETX-5 MS

Connect T1 TDM Loop to each of the TDM port in the ETX-205

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit T1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and ETX-205 | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and ETX-205 | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 5 | 2 | 2 | 1 |
|  | PW 6 | 3 | 2 | 1 |
|  | PW 7 | 1 | 3 | 1 |
|  | PW 8 | 2 | 3 | 1 |

###### Protection testing

Connect T1-TTC to ETX-205 TDM port with Protection time measurement feature.

Connect OC3 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect OC3 port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect OC3 port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

Note: Perform each test per TDM port of ETX-205

### Test Case 3: PW9 between Aggregation POP to MiTOP (T1)

Service Provisioning

- Configure ETX-5 Services PW9 according to scripts in Appendix E.3

Configure MiTOP Services PW9 according to scripts in Appendix E.3

Connect OC3-TTC to TDM port 1 on the ETX-5 MS

Connect T1 TDM Loop to the TDM port in the MiTOP

Verify the service was successfully created.

##### Test Procedure

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Set the TTC generator to transmit T1 2*10^11 pattern on each of the tributary (see table) and test no bit error occurs | No bit error occurs No frame slip | pass |
|  | Check PM screens in ETX-5 and MiTOP | No ES were counted | pass |
|  | Inject a few bits error (i.e. TSE) | A few bits error were counted | Pass |
|  | Check PM screens in ETX-5 and MiTOP | A few ES were counted | pass |

| Tributary under test |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | Service | Tributary |  |  |
|  |  | TUG3 | TUG2 | TU |
|  | PW 9 | 3 | 3 | 1 |

###### Protection testing

- Connect T1-TTC to MiTOP TDM port with Protection time measurement feature.

- Connect OC3 TDM Loop to each of the TDM port in the ETX-5 MS

| Basic TDM traffic testing |  |  |  |
| --- | --- | --- | --- |
| # | Action | Expected Result | Pass/Fail |
|  | Disconnect OC3 port 1 and connect | Few bits error, TTC >50mSec | pass |
|  | Check Status screens in ETX-5 | Port 2 is active Port 1 Standby | pass |
|  | Disconnect OC3 port 2 and connect | Few bits error, TTC >50mSec | Pass |
|  | Check Status screens in ETX-5 | Port 1 is active Port 2 Standby | pass |

Appendix A – STM1

Aggregation POP – ETX-5 Preliminary Setting

##### Factory default

admin factory-default

##### Hardware, Queues and Port Configuration

#HW Configuration #config slot 1 card-type sdh-sonet stm-1-ch-4 #config slot 1 no shutdown #*when IO card ready run script #***************************Classification_Key*************************** config port ethernet main-a/1 classification-key vlan p-bit exit all #***************************MC_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_3_level_default_MC_a/1 inherited-from q_group_3_level_default exit all #***************************BIND_QG_TO_PORTS******************************* config port ethernet main-a/1 queue-group profile q_group_3_level_default_MC_a/1 exit all #***************************Color-Mapping******************************* config qos color-map-profile dei-color-map classification dei exit all #******************************PORT_ACTIVATION************************** config port ethernet main-a/1 no shutdown exit all #***************************END******************************* #**********************END_OF_SCRIPT************************** #***************************END*******************************

##### Local Management Set Up configuration

#***************************SNMP Configuration*************************** configure system name "Aggregation_POP" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #******************************configure port svi************************** exit all configure port svi 99 router exit all #*****************************End port svi Configuration****************** #***************************Marking-Mapping******************************* config qos marking-profile "mark1" color-aware green-yellow dei mapping mark 0..7 green to 0 dei green mark 0..7 yellow to 0 dei yellow exit all #***************************End Marking-Mapping****************************** #**************** Classifier Profile Configuration ********************** config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 exit all #*************** End Classifier Profile Configuration ********************** #************************Flows_Inband_Configuration************************* config flows flow "mng_in_router" classifier "VLAN_4094" ingress-port ethernet main-a/1 egress-port svi 99 vlan-tag pop vlan no shutdown exit all config flows flow "mng_out_router" classifier "all" ingress-port svi 99 egress-port ethernet main-a/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 vlan-tag push vlan 4094 p-bit profile "mark1" tag-ether-type 0x8100 no shutdown exit all #*********************End Flows_Inband_Configuration******************************* #*************************Router _Configuration************************************ config router 1 name "Router#1" interface 1 address 172.18.170.177/24 bind svi 99 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all #********************* End Router _Configuration *******************************

##### Configure STM1 (High Level) and Protection

# ********************Define IO card****************** #config slot 1 card-type sdh-sonet stm-1-ch-4 #config slot 1 no shutdown #***************************************************** # ********************External 2Mhz clock************* config system clock station main-a/1 interface-type 2mhz no shutdown exit all config system clock domain 1 source 1 station main-a/1 quality-level prs wait-to-restore 0 clear-wait-to-restore exit all # *************************************************** # *********************** APS *********************** # *************************************************** configure protection aps 1_1and1_2 bind working sdh-sonet 1/1 bind protection sdh-sonet 1/2 no shutdown exit all # *************************************************** # ***************** Physical Ports ****************** # *************************************************** # *************************************************** # ********************** SDH ********************** # *************************************************** config port sdh-sonet 1/1 frame-type sdh no tim-action soh path vt tx-clock-source domain 1 no shutdown exit all config port sdh-sonet 1/2 frame-type sdh no tim-action soh path vt tx-clock-source domain 1 no shutdown exit all # *************************************************** # *********************** AUG *********************** # *************************************************** configure port sdh-sonet 1/1 aug 1 no shutdown configure port sdh-sonet 1/2 aug 1 no shutdown

HQ – ETX-205 Preliminary Setting

##### Factory default

admin factory-default

##### Queues Configuration

#************************ETX-203AX,ETX-205A*************************** #***************************QUEUE_GROUP******************************* exit all config qos queue-group-profile QGN1 exit all config port eth 1 queue-group profile QGN1 exit all config qos queue-group-profile QGN2 exit all config port eth 2 queue-group profile QGN2 exit all config qos queue-group-profile QGN3 exit all config port eth 3 queue-group profile QGN3 exit all config qos queue-group-profile QGN4 exit all config port eth 4 queue-group profile QGN4 exit all config qos queue-group-profile QGN5 exit all config port eth 5 queue-group profile QGN5 exit all config qos queue-group-profile QGN6 exit all config port eth 6 queue-group profile QGN6 exit all save #***************************END******************************* #**********************END_OF_SCRIPT************************** #***************************END*******************************

##### Local Management Set Up configuration

#***************************SNMP Configuration************************ configure system name "HQ" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #************************* END SNMP Configuration *********************** #*************************** Inband MNG *********************** config port svi 1 no shutdown exit all config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 config flows flow "Eth_1_svi_1" classifier "VLAN_4094" no policer vlan-tag pop vlan ingress-port ethernet 1 egress-port svi 1 queue 0 no shutdown exit all config flows flow "svi_1_Eth_1" classifier "all" no policer ingress-port svi 1 egress-port ethernet 1 queue 0 block 0/1 vlan-tag push vlan 4094 p-bit fixed 0 no shutdown exit all config router 1 name "Router#1" interface 1 address 172.18.170.178/24 bind svi 1 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all #*************************** END Inband MNG **************************

IPmux-24 – Management, TDM type and Mux Method Setting

Menu driven. Baud rate 115,200 bps.

##### Factory Default

- Root->Configuration

Configuration->System

System->Factory Default

##### TDM Type Setting

Change the IPMUX 5 TDM port type to E1:

Root->Configuration

Configuration->Physical Layer

TDM interface type – change to E1, press ‘s’, device will restart.

Note: configuration will take action after rebooting.

##### Mux method setting

Before creating any bundle

Root->Configuration

Configuration->Connection

Multiplexing->select item 3 followed by a reset to Destination Port

Note: configuration will take action after rebooting.

##### Management Configuration

Root->Configuration

Configuration->System

System->Management

Management->Host IP

DHCP Status, change to Disable and press ‘s’ to save.

Configure IP Address and Subnet Mask and press ‘s’ to save.

Configure Encapsulation VLAN 4094 for management (pBit 0) and ‘s’ to save.

Press ESC to return to Configuration screen.

Configuration->Router

Define Gate Way, and press ‘s’ to save.

Press ESC to back to Configuration Screen.

Configuration->Bridge

Change the VLAN mode to Aware, to support VLAN.

And add VLAN membership

Press A to add VLAN 4094, press ‘s’ to save.

LA-210 – Preliminary Setting

##### Factory default

admin factory-default

##### Management Configuration

configure # System Configuration system name "LA-210_TDM" exit # Port Configuration port # Service Virtual Interface- Port Configuration svi 1 name "SVI-1 " exit e1 1 name "E1-1" tx-clock-source adaptive line-type g732n exit exit # Bridge Configuration bridge 1 vlan-aware filtering port 1 pvid 4094 priority 0 exit # VLAN Configuration vlan 401 tagged-egress 6,1 exit # VLAN Configuration vlan 4094 tagged-egress 6 untagged-egress 1 exit exit router 1 interface 1 address 172.18.170.182/24 no shutdown exit interface 2 address 10.10.10.10/24 vlan 401 priority 7 name "Interface-2" no shutdown exit default-gateway address 172.18.170.1 exit management snmp snmpv3 security-to-group usm sec-name "initial" group-name "initial" no shutdown exit target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" exit all save

MiTOP – Preliminary Setting

##### MiTOP Dip Switch Roll

##### Configuration Adapter

| An optional configuration adapter (SFP-CA) is available for connecting MiTOP-E1/T1 to a PC via a USB 2.0 port. The configuration adapter is used for preliminary configuration or software download. Or use a standard L2/L3 switch with SFP sleeve |  |
| --- | --- |

.

##### Factory Default

- Remove MiTOP from sleeve.

Set DIP switch SW1 and SW2 OFF.

Plug MiTOP to sleeve (make sure Power is ON).

Wait 30 seconds.

Remove MiTOP and set Dip Switch to Normal according table above.

Default management IP address: 192.168.205.1             TDM Frame Type: E1

Use Microsoft Explorer or Google Chrome and navigate to http://192.168.205.1

##### Configuring the Host IP Parameters

Configure the host IP parameters (Configuration > System > Management > Host IP):

Disable DHCP client

Set the IP Address field: 172.18.170.180

Set the IP Mask field: 255.255.255.0

Set the Default Gateway field: IP address.

Set Host tagging to be Tagged Host VLAN ID: 4094

Host VLAN Priority: 0

Appendix B – SONET (OC-3)

##### Note: Network Elements are TDM port type E1 based, and needed to modify to T1 type.

Aggregation POP – ETX-5 Preliminary Setting

##### Factory default

admin factory-default

##### Hardware, Queues and Port Configuration

#HW Configuration #config slot 1 card-type sdh-sonet oc-3-ch-4 #config slot 1 no shutdown #*when IO card ready run script #***************************Classification_Key*************************** config port ethernet main-a/1 classification-key vlan p-bit exit all #***************************MC_QUEUE_GROUP******************************* configure qos queue-group-profile q_group_3_level_default_MC_a/1 inherited-from q_group_3_level_default exit all #***************************BIND_QG_TO_PORTS******************************* config port ethernet main-a/1 queue-group profile q_group_3_level_default_MC_a/1 exit all #***************************Color-Mapping******************************* config qos color-map-profile dei-color-map classification dei exit all #******************************PORT_ACTIVATION************************** config port ethernet main-a/1 no shutdown exit all #***************************END******************************* #**********************END_OF_SCRIPT************************** #***************************END*******************************

##### Local Management Set Up configuration

#***************************SNMP Configuration*************************** configure system name "Aggregation_POP" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #******************************configure port svi************************** exit all configure port svi 99 router exit all #*****************************End port svi Configuration****************** #***************************Marking-Mapping******************************* config qos marking-profile "mark1" color-aware green-yellow dei mapping mark 0..7 green to 0 dei green mark 0..7 yellow to 0 dei yellow exit all #***************************End Marking-Mapping****************************** #**************** Classifier Profile Configuration ********************** config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 exit all #*************** End Classifier Profile Configuration ********************** #************************Flows_Inband_Configuration************************* config flows flow "mng_in_router" classifier "VLAN_4094" ingress-port ethernet main-a/1 egress-port svi 99 vlan-tag pop vlan no shutdown exit all config flows flow "mng_out_router" classifier "all" ingress-port svi 99 egress-port ethernet main-a/1 queue-map-profile "QueueMapDefaultProfile" block 0/1 vlan-tag push vlan 4094 p-bit profile "mark1" tag-ether-type 0x8100 no shutdown exit all #*********************End Flows_Inband_Configuration******************************* #*************************Router _Configuration************************************ config router 1 name "Router#1" interface 1 address 172.18.170.177/24 bind svi 99 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all #********************* End Router _Configuration *******************************

##### Configure SONET (High Level) and Protection

# ********************Define IO card****************** #config slot 1 card-type sdh-sonet oc-3-ch-4 #config slot 1 no shutdown #***************************************************** # ********************External 2Mhz clock************* config system clock station main-a/1 interface-type 2mhz no shutdown exit all config system clock domain 1 source 1 station main-a/1 quality-level prs wait-to-restore 0 clear-wait-to-restore exit all # *************************************************** # *********************** APS *********************** # *************************************************** configure protection aps 1_1and1_2 bind working sdh-sonet 1/1 bind protection sdh-sonet 1/2 no shutdown exit all # *************************************************** # ***************** Physical Ports ****************** # *************************************************** # *************************************************** # ********************** SONET ********************** # *************************************************** config port sdh-sonet 1/1 no tim-action soh path vt tx-clock-source domain 1 no shutdown exit all config port sdh-sonet 1/2 no tim-action soh path vt tx-clock-source domain 1 no shutdown exit all # *************************************************** # *********************** HVC *********************** # *************************************************** configure port sdh-sonet 1/1 oc3 1 sts1 1 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 2 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 3 no shutdown configure port sdh-sonet 1/2 oc3 1 sts1 1 no shutdown configure port sdh-sonet 1/2 oc3 1 sts1 2 no shutdown configure port sdh-sonet 1/2 oc3 1 sts1 3 no shutdown

##### MAC Address

PWE address is published under IO SDH-SONET, change scripts to support MAC

config port sdh-sonet 1/1 show status

HQ - ETX-205 Preliminary Setting

##### Factory default

admin factory-default

##### Queues Configuration

#************************ETX-203AX,ETX-205A*************************** #***************************QUEUE_GROUP******************************* exit all config qos queue-group-profile QGN1 exit all config port eth 1 queue-group profile QGN1 exit all config qos queue-group-profile QGN2 exit all config port eth 2 queue-group profile QGN2 exit all config qos queue-group-profile QGN3 exit all config port eth 3 queue-group profile QGN3 exit all config qos queue-group-profile QGN4 exit all config port eth 4 queue-group profile QGN4 exit all config qos queue-group-profile QGN5 exit all config port eth 5 queue-group profile QGN5 exit all config qos queue-group-profile QGN6 exit all config port eth 6 queue-group profile QGN6 exit all save #***************************END******************************* #**********************END_OF_SCRIPT************************** #***************************END*******************************

##### Local Management Set Up configuration

#***************************SNMP Configuration************************ configure system name "HQ" exit all config management snmp target-params "PC_69" message-processing-model snmpv3 version usm security name "initial" level no-auth-no-priv no shutdown exit target "a" target-params "PC_69" address udp-domain 172.17.150.69 no shutdown tag-list "unmasked" trap-sync-group 1 exit config-change-notification exit all #************************* END SNMP Configuration *********************** #*************************** Inband MNG *********************** config port svi 1 no shutdown exit all config flows classifier-profile "all" match-any match all config flows classifier-profile "VLAN_4094" match-any match vlan 4094 config flows flow "Eth_1_svi_1" classifier "VLAN_4094" no policer vlan-tag pop vlan ingress-port ethernet 1 egress-port svi 1 queue 0 no shutdown exit all config flows flow "svi_1_Eth_1" classifier "all" no policer ingress-port svi 1 egress-port ethernet 1 queue 0 block 0/1 vlan-tag push vlan 4094 p-bit fixed 0 no shutdown exit all config router 1 name "Router#1" interface 1 address 172.18.170.178/24 bind svi 1 no shutdown exit static-route 0.0.0.0/0 address 172.18.170.1 exit all #*************************** END Inband MNG **************************

##### MAC Address

- PWE address is published under device-information, change scripts to support MAC

config system show device-information

##### TDM Type Setting

#*************************** DS1 - Port Configuration ************************ config port ds1 1 frame-type t1 exit all config port ds1 2 frame-type t1 exit all config port ds1 3 frame-type t1 exit all config port ds1 4 frame-type t1 exit all #*************************** END DS1 - Port Configuration **********

IPMUX-24 – Management, TDM type and Mux Method Setting

Menu driven. Baud rate 115,200 bps.

##### Factory default

- Root->Configuration

Configuration->System

System->Factory Default

##### TDM Type Setting

Change the IPMUX TDM port type to T1:

Root->Configuration

Configuration->Physical Layer

TDM interface type – change to T1, press ‘s’, device will restart.

Note: configuration will take action after rebooting.

##### Mux method setting

Before creating any bundle:

Root->Configuration

Configuration->Connection

Multiplexing->select item 3 followed by a reset to Destination Port

Note: configuration will take action after rebooting.

##### Management Configuration

Root->Configuration

Configuration->System

System->Management

Management->Host IP

DHCP Status, change to Disable and press ‘s’ to save.

Configure IP Address and Subnet Mask and press ‘s’ to save.

Configure Encapsulation VLAN 4094 for management (pBit 0) and ‘s’ to save.

Press ESC to return to Configuration screen.

Configuration->Router

Define Gate Way, and press ‘s’ to save.

Press ESC to back to Configuration Screen.

Configuration->Bridge

Change the VLAN mode to Aware, to support VLAN.

And add VLAN membership

Press A to add VLAN 4094, press ‘s’ to save.

##### MAC Address

PWEa address is published under Inventory, change scripts to support MAC

MiTOP – Preliminary Setting

##### MiTOP Dip Switch Roll

MiTOP-E1/T1 includes a 2-section DIP switch which is used for selecting one of the following operation modes of the device:

Database initialization

Normal operation

Software download

Configuration.

.

##### Configuration Adapter

| An optional configuration adapter (SFP-CA) is available for connecting MiTOP-E1/T1 to a PC via a USB 2.0 port. The configuration adapter is used for preliminary configuration or software download. Or use a standard L2/L3 switch with SFP sleeve |  |
| --- | --- |

.

##### Factory Default

- Remove MiTOP from sleeve.

Set DIP switch SW1 and SW2 OFF.

Plug MiTOP to sleeve (make sure Power is ON).

Wait 30 seconds.

Remove MiTOP and set Dip Switch to Normal according table above.

Default management IP address: 192.168.205.1             TDM Frame Type: E1

Use Microsoft Explorer, Google Chrome or other and navigate to http://192.168.205.1

Selecting the TDM Interface

Select the TDM interface type from the Physical Ports menu (Configuration > Physical Ports).

Choose the type of the MiTOP-E1/T1 TDM link T1.

##### Configuring the Host IP Parameters

Configure the host IP parameters (Configuration > System > Management > Host IP):

Disable DHCP client

Set the IP Address field: 172.18.170.180

Set the IP Mask field: 255.255.255.0

Set the Default Gateway field: IP address.

Set Host tagging to be Tagged Host VLAN ID: 4094

Host VLAN Priority: 0

##### MAC Address

PWE address is published under Monitoring->system, change scripts to support MAC

##### TDM Type Setting

Change the PW TDM type to T1

Root->Configuration

Physical Layer->Interface Type

Select T1, and press ‘SAVE’ button

Appendix C – IEEE 1588v2 Frequency Synchronization (SDH and SONET)

ETX-5 Master Configuration

#****************************** 1588-master_to_port ****************************** config port svi 202 router exit all config flows classifier-profile Vlan402 match-any match vlan 402 exit all config qos marking-profile "mark_1588" color-aware green-yellow dei mapping mark 0..7 green to 2 dei green mark 0..7 yellow to 2 dei yellow exit all #***************************** 1588 master main-a/1******************************** config flows flow 1588_m_main_a_1_svi_202 classifier Vlan402 ingress-port ethernet main-a/1 egress-port svi 202 vlan-tag pop vlan no shutdown exit all config flows flow 1588_svi_202_main_a_1 classifier all ingress-port svi 202 egress-port ethernet main-a/1 queue-map-profile QueueMapDefaultProfile block 0/2 vlan-tag push vlan 402 p-bit profile mark_1588 no shutdown exit all configure router 1 interface 4 loopback #master ip ptp address 30.30.30.30/32 no shutdown exit all configure router 1 interface 5 #master inter for int address 50.50.50.2/24 bind svi 202 no shutdown exit all configure system clock master main-a/1 ptp ip-address 30.30.30.30 sync-rate 128pps #**********************************MASTER*as slave check check****************** no shutdown exit all

ETX-205 Slave Configuration

config port svi 3 no shutdown exit all config flows classifier-profile Vlan402 match-any match vlan 402 exit all config flows flow 1588_3_input classifier "Vlan402" ingress-port ethernet 1 egress-port svi 3 vlan-tag pop vlan no shutdown exit all config flows flow 1588_3_out classifier all ingress-port svi 3 egress-port ethernet 1 queue 0 block 0/2 vlan-tag push vlan 402 p-bit fixed 2 no shutdown exit all #*****************************Router_Interface**************************** configure router 1 interface 3 address 50.50.50.3/24 bind svi 3 no management-access no shutdown exit static-route 30.30.30.30/32 address 50.50.50.2 exit peer 2 ip 30.30.30.30 exit all #**************************** 1588_slave ****************************** configure system clock domain 1 source 1 recovered 1 priority 1 quality-level ssm-based wait-to-restore 0 clear-wait-to-restore exit exit recovered 1 ptp wait-to-restore 0 master 1 peer 2 priority 1 sync grant-period 300 announce grant-period 300 delay-respond grant-period 300 quality-level type1-ssm-based no shutdown exit no shutdown exit

Appendix D – SDH – CESoPSN (UDP/IP) Services

ETX-5  IPMUX-24 Services Configuration

##### ETX-5 Services PW1-PW4

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #*********************** STM-1 ************************** configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 1 1 no shutdown configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 1 2 no shutdown configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 1 3 no shutdown configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 2 1 no shutdown # ************************************************************* # ************************************************************* # ************************* E1 CONFIG ************************* # ************************************************************* config port e1 1/1/1 line-type g732n config port e1 1/1/2 line-type g732n config port e1 1/1/3 line-type g732n config port e1 1/1/4 line-type g732n #---------------------------------------------------- # E1 Clock Port 1/1_1/2 #---------------------------------------------------- configure port e1 1/1/1 tx-clock-source domain 1 configure port e1 1/1/2 tx-clock-source domain 1 configure port e1 1/1/3 tx-clock-source domain 1 configure port e1 1/1/4 tx-clock-source domain 1 #---------------------------------------------------- # SDH Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** E1 Admin Up ******************** # *************************************************** config port e1 1/1/1 no shutdown config port e1 1/1/2 no shutdown config port e1 1/1/3 no shutdown config port e1 1/1/4 no shutdown #********************* SVI Router *********************** configure port svi 201 router name router_to_pw_udp_ip # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 1 ip 10.10.10.7 name IPMUX24 # #******************************************************* #******************* port 1/1 STM-1 pw ******************* #******************************************************* #******************* IPMUX_24*************************** config pwe pw 1 type ces-psn-data psn udp-over-ip label in 1 out 1 peer 1 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 2 type ces-psn-data psn udp-over-ip label in 33 out 33 peer 1 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 3 type ces-psn-data psn udp-over-ip label in 65 out 65 peer 1 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 4 type ces-psn-data psn udp-over-ip label in 97 out 97 peer 1 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all #******************* IPMUX_24_END *************************** # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 1 e1 1/1/1 time-slots 1..31 config cross-connect pw-tdm pw 2 e1 1/1/2 time-slots 1..31 config cross-connect pw-tdm pw 3 e1 1/1/3 time-slots 1..31 config cross-connect pw-tdm pw 4 e1 1/1/4 time-slots 1..31 # ****************************************************** # ********** PW_to_ethernet main-a/1 ********** # ****************************************************** #***************************Marking-Mapping******************************* config qos marking-profile "mark_PW" color-aware green-yellow dei mapping mark 0..7 green to 7 dei green mark 0..7 yellow to 7 dei yellow exit all config flows classifier-profile "VLAN401" match-any match vlan 401 exit all configure flows flow ETH_main-a/1_SVI_PW classifier VLAN401 ingress-port ethernet main-a/1 egress-port svi 201 vlan-tag pop vlan pm-enable no shutdown exit all configure flows flow SVI_PW_ETH_main-a/1 classifier all ingress-port svi 201 egress-port ethernet main-a/1 queue-map-profile QueueMapDefaultProfile block 0/2 vlan-tag push vlan 401 p-bit profile "mark_PW" tag-ether-type 0x8100 pm-enable no shutdown exit all #*********************************ROUTER************************ configure router 1 interface 2 loopback address 100.100.100.100/32 no shutdown exit all configure router 1 interface 3 address 10.10.10.20/24 bind svi 201 no shutdown exit all configure slot 1 shutdown bind loopback-address 100.100.100.100 no shutdown exit

##### IPMUX-24 Service PW1-PW4

###### Hot Keys

F-Forward, B-Backward -  move from between ports

N-Next, P-Previous – scroll down a configuration screen

S-Save configuration , ESC-exit previous screen

###### Configure 1-4 TDM ports

Configuration:

| Source Clock | Adaptive |
| --- | --- |
| Source Clock Quality | other/unknown |
| Line Type | Framed G.704 |

###### Configure PWE host IP and static route

- Configure PW IP 10.10.10.7 by navigating to:Root->configuration->Connection->PW Host IP

Note: in band management  by telnet would be terminated.active it again.

Add PWE Host IP to static Route

Root->Configuration

Configuration->Router

Add to static route a remote PWE IP 100.100.100.100, DG 10.10.10.7

Router->Static Route Tabel

Press ‘a’ to Add a new entry to table

###### Configure Bundles, ID (label, udp port) 1,33,65,97

Change general parameters per Bundle ID

Configuration:

| PW Type | CESoPSN |
| --- | --- |
| PSN Type | UDP/IP |

Set DS0 Bundle (cross connect)

Press ‘a’ to enable all Time Slots.

For each PWE configure:

| Destination IP Address | 100.100.100.100 |
| --- | --- |
| Connection status | Enable |
| Destination Bundle | 1,33,65,97 (per bundle) |
| TDM Bytes in Frame | 8 |
| OAM Connectivity | Disable |
| Jitter Buffer | 3 |
| Far End Type | E1 |
| VLAN Tagging | Enable |
| VLAN ID | 401 |
| VLAN Priority | 7 |

###### Bridge support PW Vlan

Root->Configuration-Bridge

Add VLAN 401 to Egress Unmodified Ports 4

Press ‘a’ to add

ETX-5  ETX-205 Services Configuration

##### ETX-5 Services PW5-PW8

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #*********************** STM-1 ************************** configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 2 2 no shutdown configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 2 3 no shutdown configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 3 1 no shutdown configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 3 2 no shutdown # ************************************************************* # ************************************************************* # ************************* E1 CONFIG ************************* # ************************************************************* config port e1 1/1/5 line-type g732n config port e1 1/1/6 line-type g732n config port e1 1/1/7 line-type g732n config port e1 1/1/8 line-type g732n #---------------------------------------------------- # E1 Clock Port 1/1_1/2 #---------------------------------------------------- configure port e1 1/1/5 tx-clock-source domain 1 configure port e1 1/1/6 tx-clock-source domain 1 configure port e1 1/1/7 tx-clock-source domain 1 configure port e1 1/1/8 tx-clock-source domain 1 #---------------------------------------------------- # SDH Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** E1 Admin Up ******************** # *************************************************** config port e1 1/1/5 no shutdown config port e1 1/1/6 no shutdown config port e1 1/1/7 no shutdown config port e1 1/1/8 no shutdown # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 2 ip 10.10.10.8 name HQ exit all # #******************************************************* #******************* port 1/1 STM-1 pw ******************* #******************************************************* #************************ HQ ******************************** config pwe pw 5 type ces-psn-data psn udp-over-ip label in 130 out 130 peer 2 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 6 type ces-psn-data psn udp-over-ip label in 131 out 131 peer 2 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 7 type ces-psn-data psn udp-over-ip label in 132 out 132 peer 2 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 8 type ces-psn-data psn udp-over-ip label in 133 out 133 peer 2 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all #************************ HQ_END ******************************** # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 5 e1 1/1/5 time-slots 1..31 config cross-connect pw-tdm pw 6 e1 1/1/6 time-slots 1..31 config cross-connect pw-tdm pw 7 e1 1/1/7 time-slots 1..31 config cross-connect pw-tdm pw 8 e1 1/1/8 time-slots 1..31

##### ETX-205 Services PW5-PW8

#*************************** SVI for PW *********************** configure port svi 2 no shutdown exit all #*************************** PW Flows ************************ configure flows classifier-profile match_all match-any match all exit all configure flows classifier-profile VLAN401 match-any match vlan 401 exit all config flows flow "p1_svi2" classifier "VLAN401" vlan-tag pop vlan ingress-port ethernet 1 egress-port svi 2 queue 0 no shutdown exit all config flows flow "svi2_p1" classifier "match_all" vlan-tag push vlan 401 p-bit fixed 0 ingress-port svi 2 egress-port ethernet 1 queue 0 block 0/1 no shutdown exit all #*************************** PW peer ************************ config peer 1 ip 100.100.100.100 name Aggregation_POP #*************************** DS1 - Port Configuration ***************** config port ds1 1 frame-type e1 exit all config port ds1 2 frame-type e1 exit all config port ds1 3 frame-type e1 exit all config port ds1 4 frame-type e1 exit all #*************************** PWE - Configuration ***************** config pwe pw 1 type ces-psn-data psn udp-over-ip peer 1 jitter-buffer 15000 label in 130 out 130 vlan priority 0 tdm-payload size 248 rate 31 psn-oos 1-bit pm-enable no shutdown exit all config pwe pw 2 type ces-psn-data psn udp-over-ip peer 1 jitter-buffer 15000 label in 131 out 131 vlan priority 0 tdm-payload size 248 rate 31 psn-oos 1-bit pm-enable no shutdown exit all config pwe pw 3 type ces-psn-data psn udp-over-ip peer 1 jitter-buffer 15000 label in 132 out 132 vlan priority 0 tdm-payload size 248 rate 31 psn-oos 1-bit pm-enable no shutdown exit all config pwe pw 4 type ces-psn-data psn udp-over-ip peer 1 jitter-buffer 15000 label in 133 out 133 vlan priority 0 tdm-payload size 248 rate 31 psn-oos 1-bit pm-enable no shutdown exit all #******************** e1 - Port Configuration *************** config port e1 1 no shutdown tx-clock-source domain 1 line-type g732n exit all config port e1 2 no shutdown tx-clock-source domain 1 line-type g732n exit all config port e1 3 no shutdown tx-clock-source domain 1 line-type g732n exit all config port e1 4 no shutdown tx-clock-source domain 1 line-type g732n exit all #******************** Cross-connect - Configuration *********** config cross-connect pw-tdm pw 1 e1 1 time-slots 1..31 config cross-connect pw-tdm pw 2 e1 2 time-slots 1..31 config cross-connect pw-tdm pw 3 e1 3 time-slots 1..31 config cross-connect pw-tdm pw 4 e1 4 time-slots 1..31 exit all #******************** Router interface *********** config router 1 name "Router#1" interface 2 address 10.10.10.8/24 bind svi 2 no management-access no shutdown exit static-route 100.100.100.100/32 address 10.10.10.20 exit all

ETX-5   MiTOP Services Configuration

##### ETX-5 Service PW9

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #*********************** STM-1 ************************** configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 3 3 no shutdown # ************************************************************* # ************************************************************* # ************************* E1 CONFIG ************************* # ************************************************************* config port e1 1/1/9 line-type g732n #---------------------------------------------------- # E1 Clock Port 1/1_1/2 #---------------------------------------------------- configure port e1 1/1/9 tx-clock-source domain 1 #---------------------------------------------------- # SDH Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** E1 Admin Up ******************** # *************************************************** config port e1 1/1/9 no shutdown # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 3 ip 10.10.10.9 name MiTOP # #******************************************************* #******************* port 1/1 STM-1 pw ******************* #******************************************************* #************************ MiTOP ******************************** config pwe pw 9 type ces-psn-data psn udp-over-ip label in 134 out 134 peer 3 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all #******************** MiTOP_END ******************************** # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 9 e1 1/1/9 time-slots 1..31

##### MiTOP Service PW9

###### Hot Keys

Previous menu – go back one step menu

Refresh – reload changed values

Save – save changed parameter

###### Configure TDM port

- Root->Configuration->Physical Ports

Press on ‘E1’ and configure, press ‘SAVE’ button

Configuration:

| TX Clock Source | Adaptive Clock |
| --- | --- |
| Line Code | HDB3 |
| RX sensitivity | Long Haul |
| Line Type | E1 G.732N |

###### Configure PEER IP

Root->Configuration->Multi service over PSN

Multi service over PSN ->Peer

Change configuration and press ‘SAVE’ button

Configuration:

| Peer IP Address | 100.100.100.100 |
| --- | --- |
| Next Hope | 10.10.10.20 |

###### General PW Configuration

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW->General Configuration)

Configuration:

| Source IP Address | 10.10.10.9 |
| --- | --- |
| Multiplexing | Destination |
| OAM | Disable |
| Out PW label | 134 |
| In PW label | 134 |

###### PSN parameters

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW->PSN parameters)

Configuration:

| Reorder Packets | Enable |
| --- | --- |
| RTP Header | Disable |
| TOS | 0 |
| VLAN tagging | Enable |
| VLAN priority | 0 |
| VLAN ID | 401 |

###### Service parameters

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW->Service parameters)

Configuration:

| Payload size | 8 |
| --- | --- |
| Jitter buffer | 8000 |

###### PW configuration

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW)

Configuration:

| Connection status | Enable |
| --- | --- |
| Trail Mode | Terminated |

Press ‘SAVE’ button.

ETX-5  LA-210 Services Configuration

##### ETX-5 Service PW10

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #*********************** STM-1 ************************** configure port sdh-sonet 1/1 aug 1 tug3 1 vc12 4 1 no shutdown # ************************************************************* # ************************************************************* # ************************* E1 CONFIG ************************* # ************************************************************* config port e1 1/1/10 line-type g732n #---------------------------------------------------- # E1 Clock Port 1/1_1/2 #---------------------------------------------------- configure port e1 1/1/10 tx-clock-source domain 1 #---------------------------------------------------- # SDH Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** E1 Admin Up ******************** # *************************************************** config port e1 1/1/10 no shutdown # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 4 ip 10.10.10.10 name LA210 # #******************************************************* #******************* port 1/1 STM-1 pw ******************* #******************************************************* #********************** LA210_E1 ******************************** config pwe pw 10 type ces-psn-data psn udp-over-ip label in 135 out 135 peer 4 tdm-payload size 248 rate 31 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 10 e1 1/1/10 time-slots 1..31

##### LA-210 Service PW10

config router 1 peer 1 ip 100.100.100.100 name "Peer-1" static-route 100.100.100.100/32 address 10.10.10.20 exit all config pwe pw 1 type ces-psn-data psn udp-over-ip manually name "Put your string here" peer 1 label in 135 out 135 tdm-payload size 248 rate 31 jitter-buffer 100000 udp-mux-method dst-port no shutdown exit exit all configure system clock recovered 1 adaptive pw 1 exit all echo "Cross Connect" # Cross Connect configure cross-connect pw-tdm pw 1 e1 1 time-slots 1..31 exit

Appendix E – SONET – SATOP (MEF 8)

ETX-5    IPmux-24 Services Configuration

##### ETX-5 Services PW1-PW4

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #**************************** STS1-1 ************************** configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 1 1 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 2 1 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 3 1 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 4 1 no shutdow # ************************************************************* # ************************* T1 CONFIG ************************* # ************************************************************* config port t1 1/1/1 line-type unframed config port t1 1/1/2 line-type unframed config port t1 1/1/3 line-type unframed config port t1 1/1/4 line-type unframed #---------------------------------------------------- # SONET Port 1/1 #---------------------------------------------------- configure port t1 1/1/1 tx-clock-source domain 1 configure port t1 1/1/2 tx-clock-source domain 1 configure port t1 1/1/3 tx-clock-source domain 1 configure port t1 1/1/4 tx-clock-source domain 1 #---------------------------------------------------- # SONET Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** T1 Admin Up ******************** # *************************************************** config port t1 1/1/1 no shutdown config port t1 1/1/2 no shutdown config port t1 1/1/3 no shutdown config port t1 1/1/4 no shutdown # ****************** PW SVI ******************** configure port svi 201 pw exit all # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 1 mac 00-20-D2-26-46-CD name IPMUX24 exit all # #******************************************************* #******************* port 3/1 oc3 pw ******************* #******************************************************* #******************* IPMUX_24*************************** config pwe pw 1 type t1satop psn ethernet label in 1 out 1 peer 1 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 2 type t1satop psn ethernet label in 33 out 33 peer 1 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 3 type t1satop psn ethernet label in 65 out 65 peer 1 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 4 type t1satop psn ethernet label in 97 out 97 peer 1 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all #******************* IPMUX_24_END *************************** # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 1 t1 1/1/1 config cross-connect pw-tdm pw 2 t1 1/1/2 config cross-connect pw-tdm pw 3 t1 1/1/3 config cross-connect pw-tdm pw 4 t1 1/1/4 # ****************************************************** # ********** PW_to_ethernet main-a/1 ********** # ****************************************************** configure port svi 1 bridge exit all configure port svi 2 bridge exit all config bridge 1 port 1 bind svi 1 no shutdown exit port 2 bind svi 2 no shutdown exit vlan 401 tagged-egress 1..2 exit all config flows classifier-profile "VLAN401" match-any match vlan 401 exit all configure flows flow ETH_main-a/1_SVI_Bridge classifier VLAN401 ingress-port ethernet main-a/1 egress-port svi 2 no shutdown exit all configure flows flow SVI_Bridge_SVI_PW classifier VLAN401 ingress-port svi 1 egress-port svi 201 vlan-tag pop vlan no shutdown exit all configure flows flow SVI_PW_SVI_Bridge classifier all ingress-port svi 201 egress-port svi 1 vlan-tag push vlan 401 p-bit fixed 0 pm-enable no shutdown exit all configure flows flow SVI_Bridge_ETH_main-a/1 classifier VLAN401 ingress-port svi 2 egress-port ethernet main-a/1 queue-map-profile QueueMapDefaultProfile block 0/2 pm-enable no shutdown exit all

##### IPMUX-24 Service PW1-PW4

###### Configure 1-4 TDM ports

Hot Keys

F-Forward, B-Backward -  move from between ports 1-4

N-Next, P-Previous – scroll down a configuration screen

S-Save configuration , ESC-exit previous screen

Configuration:

| Source Clock | Adaptive |
| --- | --- |
| Source Clock Quality | other/unknown |
| Line Type | Unframed |

###### Configure PWE host IP and static route

- Configure PW IP 10.10.10.7 by navigating to:Root->configuration->Connection->PW Host IP

Note: in band management  by telnet would be terminated.active it again.

Add PWE Host IP to static Route

Root->Configuration

Configuration->Router

Add to static route a remote PWE IP 100.100.100.100, DG 10.10.10.7

Router->Static Route Tabel

Press ‘a’ to Add a new entry to table

###### Configure Bundles, ID (label, udp port) 1,33,65,97

Change general parameters per Bundle ID

Configuration:

| PW Type | SAToP |
| --- | --- |
| PSN Type | MAC/ETH |

For each PWE configure:

| Destination IP Address | 20.20.20.20 |
| --- | --- |
| Connection status | Enable |
| Destination Bundle | 1,33,65,97 (per bundle) |
| Next Hop Type | MAC |
| Next Hop MAC Address | 00-20-D2-F5-82-08 |
| TDM Bytes in Packet | 192 |
| Jitter Buffer | 3 |
| VLAN Tagging | Enable |
| VLAN ID | 401 |
| VLAN Priority | 7 |

###### Bridge support PW Vlan

Root->Configuration-Bridge

Add VLAN 401 to Egress Unmodified Ports 4

Press ‘a’ to add

ETX-5  ETX-205 Services Configuration

##### ETX-5 Services PW5-PW8

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #**************************** STS1-1 ************************** configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 5 1 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 6 1 no shutdown configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 7 1 no shutdown # ************************************************************* # ************************* T1 CONFIG ************************* # ************************************************************* config port t1 1/1/5 line-type unframed config port t1 1/1/6 line-type unframed config port t1 1/1/7 line-type unframed config port t1 1/1/8 line-type unframed #---------------------------------------------------- # SONET Port 1/1 #---------------------------------------------------- configure port t1 1/1/5 tx-clock-source domain 1 configure port t1 1/1/6 tx-clock-source domain 1 configure port t1 1/1/7 tx-clock-source domain 1 configure port t1 1/1/8 tx-clock-source domain 1 #---------------------------------------------------- # SONET Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** T1 Admin Up ******************** # *************************************************** config port t1 1/1/5 no shutdown config port t1 1/1/6 no shutdown config port t1 1/1/7 no shutdown config port t1 1/1/8 no shutdown # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 2 mac 00-20-D2-51-CE-D2 name HQ exit all # #******************************************************* #******************* port 3/1 oc3 pw ******************* #******************************************************* #************************ HQ ******************************** config pwe pw 5 type t1satop psn ethernet label in 130 out 130 peer 2 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 6 type t1satop psn ethernet label in 131 out 131 peer 2 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 7 type t1satop psn ethernet label in 132 out 132 peer 2 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all config pwe pw 8 type t1satop psn ethernet label in 133 out 133 peer 2 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all #************************ HQ_END ******************************** # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 5 t1 1/1/5 config cross-connect pw-tdm pw 6 t1 1/1/6 config cross-connect pw-tdm pw 7 t1 1/1/7 config cross-connect pw-tdm pw 8 t1 1/1/8

##### ETX-205 Services PW5-PW8

configure port svi 2 no shutdown exit all configure flows classifier-profile match_all match-any match all exit all configure flows classifier-profile VLAN401 match-any match vlan 401 exit all config flows flow "p1_svi2" classifier "VLAN401" vlan-tag pop vlan ingress-port ethernet 1 egress-port svi 2 queue 0 no shutdown exit all config flows flow "svi2_p1" classifier "match_all" vlan-tag push vlan 401 p-bit fixed 0 ingress-port svi 2 egress-port ethernet 1 queue 0 block 0/1 no shutdown exit all config peer 1 mac 00-20-D2-F5-82-08 name Aggregation_POP # DS1 - Port Configuration config port ds1 1 frame-type t1 exit all config port ds1 2 frame-type t1 exit all config port ds1 3 frame-type t1 exit all config port ds1 4 frame-type t1 exit all # PWE - Configuration config pwe pw 1 type t1satop psn ethernet peer 1 label in 130 out 130 vlan priority 0 tdm-payload size 192 psn-oos 1-bit egress-port svi 2 pm-enable no shutdown exit all config pwe pw 2 type t1satop psn ethernet peer 1 label in 131 out 131 vlan priority 0 tdm-payload size 192 psn-oos 1-bit egress-port svi 2 pm-enable no shutdown exit all config pwe pw 3 type t1satop psn ethernet peer 1 label in 132 out 132 vlan priority 0 tdm-payload size 192 psn-oos 1-bit egress-port svi 2 pm-enable no shutdown exit all config pwe pw 4 type t1satop psn ethernet peer 1 label in 133 out 133 vlan priority 0 tdm-payload size 192 psn-oos 1-bit egress-port svi 2 pm-enable no shutdown exit all config cross-connect pw-tdm pw 1 t1 1 config cross-connect pw-tdm pw 2 t1 2 config cross-connect pw-tdm pw 3 t1 3 config cross-connect pw-tdm pw 4 t1 4 exit all # T1 - Port Configuration config port t1 1 no shutdown tx-clock-source domain 1 line-type unframed exit all config port t1 2 no shutdown tx-clock-source domain 1 line-type unframed exit all config port t1 3 no shutdown tx-clock-source domain 1 line-type unframed exit all config port t1 4 no shutdown tx-clock-source domain 1 line-type unframed exit all

ETX-5   MiTOP Services Configuration

##### ETX-5 Service PW9

# *************************************************** # *********************** LVC *********************** # *************************************************** #---------------------------------------------------- # Port 1/1 #---------------------------------------------------- #**************************** STS1-1 ************************** configure port sdh-sonet 1/1 oc3 1 sts1 1 vt1-5 1 2 no shutdown # ************************************************************* # ************************* T1 CONFIG ************************* # ************************************************************* config port t1 1/1/9 line-type unframed #---------------------------------------------------- # SONET Port 1/1 #---------------------------------------------------- configure port t1 1/1/9 tx-clock-source domain 1 #---------------------------------------------------- # SONET Port 1/1 #---------------------------------------------------- # *************************************************** # ****************** T1 Admin Up ******************** # *************************************************** config port t1 1/1/9 no shutdown # *************************************************** # *************** Peer configuration***************** # *************************************************** config peer 3 mac 00-20-D2-42-84-3F name MiTOP exit all # #******************************************************* #******************* port 3/1 oc3 pw ******************* #******************************************************* #************************ MiTOP ******************************** config pwe pw 9 type t1satop psn ethernet label in 134 out 134 peer 3 egress-port svi 201 tdm-payload size 192 jitter-buffer 10000 no oam psn-oos 1-bit no shutdown exit all #******************** MiTOP_END ******************************** # *************************************************** # ****************** PW assignment ****************** # *************************************************** config cross-connect pw-tdm pw 9 t1 1/1/9

##### MiTOP Service PW9

###### Hot Keys

Previous menu – go back one step menu

Refresh – reload changed values

Save – save changed parameter

###### Configure TDM port

Root->Configuration->Physical Ports

Press on ‘T1’ and configure, press ‘SAVE’ button

Configuration:

| TX Clock Source | Adaptive Cloc |
| --- | --- |
| Line Code | B8ZS |
| RX sensitivity | Long Haul |
| Line Type | T1 Unframed |
| Line Length | DSU:0 – 133 ft |

###### Configure PEER IP

Root->Configuration->Multi service over PSN

Multi service over PSN ->Peer

Change configuration and press ‘SAVE’ button

Configuration:

| Peer IP Address | 20.20.20.20 |
| --- | --- |
| Next Hope | 20.20.20.1 |
| Peer MAC Address | 00-20-D2-F5-82-08 |

###### General PW Configuration

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW->General Configuration)

Configuration:

| Source IP Address | 10.10.10.9 |
| --- | --- |
| PSN Type | MEF |
| Multiplexing | Destination |
| OAM | Disable |
| Out PW label | 134 |
| In PW label | 134 |

###### PSN parameters

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW->PSN parameters)

Configuration:

| Reorder Packets | Enable |
| --- | --- |
| RTP Header | Disable |
| TOS | 0 |
| VLAN tagging | Enable |
| VLAN priority | 0 |
| VLAN ID | 401 |

###### Service parameters

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW->Service parameters)

Configuration:

| Payload size | 96 |
| --- | --- |
| Jitter buffer | 8000 |

###### PW configuration

Press ‘Previous Menu’

(Root->Configuration->Multi service over PSN->PW)

Configuration:

| Connection status | Enable |
| --- | --- |
| Trail Mode | Terminated |

Press ‘SAVE’ button.